XIANGQI ROBOT

SOLUTION & RECOVERY CATALOG

Tài liệu xử lý trạng thái, lỗi và recovery theo State ID

Phiên bản 1.0
Ngày: 15/09/2026

Companion document của State & Scenario Reference Catalog v1.0
Mỗi ST-... được ánh xạ 1-1 tới SOL-ST-...

Lưu ý: đây là thiết kế giải pháp kỹ thuật đề xuất. Các ngưỡng mm, confidence, retry và motion cụ thể phải được chốt bằng benchmark/tolerance thực tế. Vision/application logic không thay thế E-stop hoặc chức năng safety-rated của robot.

# 1. Mục tiêu và quan hệ với State Catalog

Mục tiêu: Biến từng State ID trong State & Scenario Reference Catalog thành một procedure xử lý có thể triển khai, test và debug.

Nguyên tắc tham chiếu: State Catalog mô tả hiện tượng; tài liệu này mô tả root cause có thể có, evidence cần thu, detection/decision logic, safety action, recovery, retry/abort, verification và test.

ST-P70-A02  <->  SOL-ST-P70-A02
State: Drop between intersections / off-grid
Solution: detect loose piece -> localize -> recovery-zone gate -> repick -> verify -> continue/abort

## Tài liệu nguồn

Requirements & Measurable Metrics Specification v1.0

Implementation Roadmap & Execution Guide v1.0

State & Scenario Reference Catalog v1.0

## 1.1 Phân biệt phần nguồn và phần đề xuất

Source-derived: State ID, tên state, type, severity, ý nghĩa, evidence cơ bản, requirement/scenario liên quan được giữ nguyên từ State Catalog.

Proposed solution design: Root-cause tree, procedure xử lý, retry policy, module ownership và test chi tiết dưới đây là thiết kế triển khai đề xuất, cần benchmark trước khi freeze.

# 2. Recovery Doctrine - luật chung cho toàn hệ thống

| Rule | Ý nghĩa triển khai |
| --- | --- |
| G1 - No silent success | Robot/SDK trả success không đồng nghĩa physical action thành công. Mọi PICK/PLACE/CAPTURE phải có post-condition verification. |
| G2 - UNKNOWN is a valid state | Nếu evidence không đủ, trả UNKNOWN/AMBIGUOUS và RESCAN/HUMAN_REQUIRED; không ép model chọn class gần nhất. |
| G3 - No commit before verify | P100 chỉ được commit khi P90 VERIFIED và discrepancy_set rỗng. |
| G4 - Geometry versioning | Board geometry và robot registration phải có version/validity. Board/camera move vượt tolerance lập tức invalidate mapping. |
| G5 - Single authoritative state writer | Chỉ StateCoordinator/TransactionManager được mutate ExpectedBoard, CurrentAction và state_version. |
| G6 - Retry phải sửa được nguyên nhân | Không retry cùng action với cùng input/pose nếu chưa re-observe/re-localize hoặc sửa cause. |
| G7 - Retry có giới hạn | Baseline: perception rescan tối đa 2 vòng; physical manipulation retry mặc định tối đa 1 vòng sau cause-correction. Các số này là provisional và phải benchmark. |
| G8 - Single anomaly before auto recovery | Auto recovery ưu tiên lỗi đơn, có vị trí và identity rõ. Multiple independent discrepancies/overlap/unknown -> safe stop hoặc HUMAN_REQUIRED. |
| G9 - Safety dominates game logic | Human/obstacle/collision/native safety fault luôn thắng mọi yêu cầu tiếp tục ván cờ. |
| G10 - Full resync after abnormal recovery | Sau recovery phải verify board semantic + physical geometry + CurrentAction trước khi commit/next turn. |

## 2.1 Escalation ladder

NORMAL
  -> continue workflow

UNCERTAIN / TRANSIENT
  -> WAIT / RESCAN
  -> if resolved: return to origin state
  -> if persistent: classify anomaly or HUMAN_REQUIRED

SINGLE RECOVERABLE ANOMALY
  -> stop current phase
  -> diagnose
  -> controlled recovery/retry
  -> full verify
  -> continue only if VERIFIED

CRITICAL / MULTIPLE / UNSAFE
  -> inhibit motion
  -> safe terminal / HUMAN_REQUIRED
  -> full reconcile before resume

# 3. Standard Procedures dùng chung

PROC-01 Fresh Rescan - Discard stale buffer -> wait scene stable -> capture fresh burst -> geometry check -> board inference -> temporal consensus -> return confidence + evidence.

PROC-02 Geometry Revalidation - Detect/confirm board corners + orientation -> compute homography -> reprojection check -> issue new geometry_version -> invalidate/refresh robot registration -> full board rescan.

PROC-03 Board Reconcile - Freeze commit -> compare ExpectedBoard/ObservedBoard/CurrentAction/event log -> classify discrepancy -> only update authoritative state through coordinator -> verify equality.

PROC-04 Controlled Physical Retry - Verify scene SAFE -> fresh target localization -> check mapping/tolerance -> increment attempt -> execute one retry -> post-action verify -> escalate on repeat failure.

PROC-05 Loose-piece Recovery - Confirm missing moving_piece context -> detect free-space object -> reject overlap/ambiguous -> continuous localization -> recovery-zone/reachability gate -> repick -> verify -> place intended destination -> full verify.

PROC-06 Wrong-piece Rollback - Detect unexpected disappeared cell -> stop planned move -> if single wrong piece securely held and original cell known/clear, return once -> verify full board -> otherwise HUMAN_REQUIRED.

PROC-07 Safe Terminal Entry - Inhibit autonomous motion -> preserve native safety state -> persist CurrentAction/board/evidence -> expose State ID + next manual steps -> require explicit resume and full validation.

PROC-08 Restart Reconcile - Load persisted action/version -> no motion -> fresh physical observation -> compare pre/post-condition hypotheses -> recover only if unique validated path -> otherwise HUMAN_REQUIRED.

PROC-09 Full Post-action Verify - Robot clear -> fresh stable burst -> semantic board compare -> source/destination check -> loose-object check -> physical geometry check -> confidence gate -> VERIFIED or explicit failure State ID.

# 4. Quick Resolution Index

Search chính xác SOL-ST-... trong Word để đi thẳng tới procedure chi tiết. Bảng dưới ưu tiên các Transient/Anomaly/Terminal state.

| Solution Key | State | Type/Severity | Primary response |
| --- | --- | --- | --- |
| SOL-ST-P00-T01 | Initial board not yet trusted | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P00-A01 | Camera unavailable at startup | ANOMALY / Critical | NO MOTION -> camera recovery |
| SOL-ST-P00-A02 | Board geometry/orientation invalid | ANOMALY / Critical | NO MOTION -> recalibrate/revalidate |
| SOL-ST-P00-A03 | Initial observed board mismatch | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P00-A04 | Persisted action incomplete after restart | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P10-T01 | Human hand present / board occluded | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P10-T02 | Robot self-occlusion | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P10-T03 | Perception uncertain but scene otherwise stable | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P10-A01 | Board moved relative to calibration | ANOMALY / Critical | NO MOTION -> recalibrate/revalidate |
| SOL-ST-P10-A02 | Camera feed stale/frozen | ANOMALY / Critical | NO MOTION -> camera recovery |
| SOL-ST-P10-A03 | Expected vs observed desynchronization before turn | ANOMALY / Major | FREEZE -> reconcile |
| SOL-ST-P20-T01 | Human touches piece then returns it | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P20-T02 | Human lifts piece but move not finished | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P20-A01 | Human illegal move | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P20-A02 | Two or more pieces changed | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P20-A03 | Pieces swapped / replaced | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P20-A04 | Piece added or removed without legal move | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P20-A05 | Human places piece off-grid | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P20-A06 | Occlusion never resolves / move cannot be observed | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P30-T01 | Multiple transition hypotheses | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P30-T02 | Legal move but perception confidence insufficient | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P30-A01 | Observed transition remains illegal after stable rescan | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P30-A02 | ExpectedBoard and observed transition conflict | ANOMALY / Major | FREEZE -> reconcile |
| SOL-ST-P30-A03 | Board orientation interpretation inconsistent | ANOMALY / Critical | NO MOTION -> recalibrate/revalidate |
| SOL-ST-P40-T01 | Target localization available but uncertainty near limit | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P40-A01 | Target outside robot workspace / auto-recovery zone | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P40-A02 | Board-to-robot mapping invalid | ANOMALY / Critical | NO MOTION -> recalibrate/revalidate |
| SOL-ST-P40-A03 | Scene unsafe or human present before motion | ANOMALY / Critical | INHIBIT / SAFE STOP |
| SOL-ST-P50-A01 | Missed captured-piece grasp | ANOMALY / Major | VERIFY_FAIL -> relocalize -> retry once |
| SOL-ST-P50-A02 | Captured piece dropped during removal | ANOMALY / Major | STOP PHASE -> locate/recover or human |
| SOL-ST-P50-A03 | Wrong piece removed | ANOMALY / Critical | NO COMMIT -> rollback/reconcile |
| SOL-ST-P50-A04 | Captured piece not cleared from board/bin path | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P60-T01 | Piece offset but still inside safe grasp tolerance | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P60-A01 | Miss grasp | ANOMALY / Major | VERIFY_FAIL -> relocalize -> retry once |
| SOL-ST-P60-A02 | Wrong adjacent piece picked | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P60-A03 | Two pieces picked / stuck together | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P60-A04 | Pick pushes or shifts neighboring piece | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P60-A05 | Gripper actuator fails to open/close as commanded | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P60-A06 | Unsafe contact / Z-height error during pick | ANOMALY / Critical | INHIBIT / SAFE STOP |
| SOL-ST-P70-A01 | Drop onto an empty valid intersection | ANOMALY / Major | STOP PHASE -> locate/recover or human |
| SOL-ST-P70-A02 | Drop between intersections / off-grid | ANOMALY / Major | STOP PHASE -> locate/recover or human |
| SOL-ST-P70-A03 | Drop onto occupied intersection / another piece | ANOMALY / Critical | STOP PHASE -> locate/recover or human |
| SOL-ST-P70-A04 | Drop outside board but inside recovery zone | ANOMALY / Major | STOP PHASE -> locate/recover or human |
| SOL-ST-P70-A05 | Drop outside camera/workspace | ANOMALY / Critical | NO MOTION -> camera recovery |
| SOL-ST-P70-A06 | Collision causes collateral piece movement | ANOMALY / Critical | INHIBIT / SAFE STOP |
| SOL-ST-P70-T01 | Carry state uncertain due to occlusion | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P80-A01 | Piece placed with excessive offset | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P80-A02 | Piece sticks to gripper after release | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P80-A03 | Piece tips / lies down | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P80-A04 | Destination unexpectedly occupied | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P80-A05 | Place pushes neighboring piece | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P80-A06 | Piece released at wrong logical cell | ANOMALY / Critical | NO COMMIT -> rollback/reconcile |
| SOL-ST-P90-T01 | Verification temporarily blocked | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P90-A01 | Destination empty after supposed place | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P90-A02 | Source still occupied after supposed pick/move | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P90-A03 | Wrong cell/piece changed | ANOMALY / Critical | NO COMMIT -> rollback/reconcile |
| SOL-ST-P90-A04 | Multiple independent discrepancies | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P90-A05 | Semantic board correct but physical geometry unsafe | ANOMALY / Major | NO MOTION -> recalibrate/revalidate |
| SOL-ST-P90-A06 | Verification confidence insufficient | ANOMALY / Major | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P100-A01 | Commit attempted with unresolved discrepancy | ANOMALY / Critical | FREEZE -> reconcile |
| SOL-ST-P100-A02 | State update race / bypass coordinator | ANOMALY / Critical | FREEZE -> reconcile |
| SOL-ST-P110-T01 | Recovery result ambiguous | TRANSIENT / AMBIGUOUS / Warning | WAIT/RESCAN -> no commit |
| SOL-ST-P110-X01 | HUMAN_REQUIRED | TERMINAL / SAFE STOP / Terminal | SAFE STOP / HUMAN |
| SOL-ST-P110-X02 | CAMERA_INVALID | TERMINAL / SAFE STOP / Terminal | SAFE STOP / HUMAN |
| SOL-ST-P110-X03 | ROBOT_ERROR | TERMINAL / SAFE STOP / Terminal | SAFE STOP / HUMAN |
| SOL-ST-P110-X04 | UNSAFE_STOP | TERMINAL / SAFE STOP / Terminal | SAFE STOP / HUMAN |
| SOL-ST-P110-X05 | PIECE_LOST | TERMINAL / SAFE STOP / Terminal | SAFE STOP / HUMAN |
| SOL-ST-P120-A01 | Software crash mid-action | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |
| SOL-ST-P120-A02 | Persisted state and physical board disagree after restart | ANOMALY / Critical | STOP CURRENT PHASE -> diagnose/recover |

# 5. P00 — Startup & Restore

Mục tiêu phase: Khởi động/restore nhưng chưa cho phép motion cho tới khi camera, geometry và board state đủ tin cậy.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P00-N01 SOL-ST-P00-N01 | NORMAL | System boot healthy |
| ST-P00-N02 SOL-ST-P00-N02 | NORMAL | Restore known mid-game state |
| ST-P00-T01 SOL-ST-P00-T01 | TRANSIENT / AMBIGUOUS | Initial board not yet trusted |
| ST-P00-A01 SOL-ST-P00-A01 | ANOMALY | Camera unavailable at startup |
| ST-P00-A02 SOL-ST-P00-A02 | ANOMALY | Board geometry/orientation invalid |
| ST-P00-A03 SOL-ST-P00-A03 | ANOMALY | Initial observed board mismatch |
| ST-P00-A04 SOL-ST-P00-A04 | ANOMALY | Persisted action incomplete after restart |

## SOL-ST-P00-N01 — System boot healthy

| State source | ST-P00-N01 — System boot healthy |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Ứng dụng, camera, model/config và state store khởi tạo được; chưa có bất thường rõ ràng. |
| Source evidence | Camera live; config/weights đọc được; không có lỗi critical. |
| Requirements | R4,R6,R11,R12,R13 |
| Scenario | S20 nếu restart midgame |

### Normal handling contract

Xác nhận evidence của state: Camera live; config/weights đọc được; không có lỗi critical..

Chỉ cho phép transition sang: READY_FOR_INITIAL_OBSERVE.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P00-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P00-N02 — Restore known mid-game state

| State source | ST-P00-N02 — Restore known mid-game state |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Hệ thống khôi phục được ván đang chơi từ persisted state và chuẩn bị đối chiếu với camera. |
| Source evidence | ExpectedBoard + CurrentAction/version được đọc; chưa commit thêm. |
| Requirements | R6,R13 |
| Scenario | S20 |

### Normal handling contract

Xác nhận evidence của state: ExpectedBoard + CurrentAction/version được đọc; chưa commit thêm..

Chỉ cho phép transition sang: P10 observation/reconcile.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P00-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P00-T01 — Initial board not yet trusted

| State source | ST-P00-T01 — Initial board not yet trusted |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Camera đã có hình nhưng chưa đủ bằng chứng để coi board state là đáng tin. |
| Source evidence | ObservedBoard còn UNKNOWN/low confidence hoặc geometry chưa validated. |
| Requirements | R1,R4,R12,R13 |
| Scenario | Startup transient |

### 1. Possible root causes

Camera vừa khởi động nên exposure/focus chưa ổn định.

Board geometry chưa được validate hoặc orientation chưa xác định.

Board recognizer trả nhiều UNKNOWN/low-confidence cell trong các frame đầu.

### 2. Evidence cần để phân biệt root cause

Burst 5-10 frame mới nhất với frame_id/timestamp tăng đều.

geometry_valid, orientation_confidence, reprojection residual.

Tỷ lệ UNKNOWN và confidence theo 90 intersection.

### 3. Detection / decision logic

Logic đề xuất: Chỉ chuyển sang TRUSTED khi camera fresh, geometry valid và board state ổn định qua nhiều frame; không dùng một frame đầu tiên.

### 4. Immediate safety / state action

Giữ robot disabled; không commit board.

Chờ settling time ngắn và lấy burst mới.

### 5. Automatic recovery / operator procedure

Chạy lại geometry check; lấy burst mới; dùng temporal consensus.

Nếu vẫn không đạt sau giới hạn rescan, route sang camera/geometry anomaly tương ứng.

### 6. Retry / abort limit

Policy: Cho phép tối đa 2 chu kỳ rescan trong một lần startup; không loop vô hạn.

### 7. Verification sau recovery

ObservedBoard ổn định giữa các frame.

Geometry valid; không còn critical UNKNOWN ở cell liên quan.

Có state_version ban đầu nhưng chưa có physical action.

### 8. Logging bắt buộc

state_id=ST-P00-T01; solution_ref=SOL-ST-P00-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Startup với camera vừa bật; exposure thay đổi.

Che một góc bàn trong frame đầu rồi bỏ che.

PASS nếu không motion/commit trước khi board trusted.

### 10. Module ownership / implementation hooks

CameraHealthMonitor

GeometryManager

BoardRecognizer

ConfidencePolicy

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P00-A01 — Camera unavailable at startup

| State source | ST-P00-A01 — Camera unavailable at startup |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Không mở được camera hoặc không có frame hợp lệ. |
| Source evidence | No frame / device error / stale feed. |
| Requirements | R11,R12,R13 |
| Scenario | S14 |

### 1. Possible root causes

Camera chưa kết nối/USB lỗi/quyền truy cập bị chiếm.

Sai camera index/device path.

Driver/capture backend lỗi.

Camera mở được nhưng chỉ trả frame rỗng/stale.

### 2. Evidence cần để phân biệt root cause

Open-camera error code; device list.

Frame timestamp/frame_id; kiểm tra độ tươi frame.

Một ảnh sample nếu có; camera settings hiện hành.

### 3. Detection / decision logic

Logic đề xuất: Camera chỉ được coi AVAILABLE khi nhận được nhiều frame mới liên tiếp; open() thành công nhưng frame stale vẫn là fail.

### 4. Immediate safety / state action

Không cho robot motion.

Đặt camera_state=INVALID và giữ CurrentAction không thay đổi.

### 5. Automatic recovery / operator procedure

Thử reopen capture một lần sau khi release handle.

Nếu reconnect thành công: warm-up -> fresh-frame check -> geometry validation -> board rescan.

Nếu thất bại: chuyển ST-P110-X02 CAMERA_INVALID.

### 6. Retry / abort limit

Policy: Tối đa 1 automatic reconnect cycle; lần tiếp theo cần operator để tránh reconnect loop.

### 7. Verification sau recovery

Frame_id/timestamp tăng; ảnh không blank.

Geometry được validate lại.

ObservedBoard đủ tin cậy trước khi resume.

### 8. Logging bắt buộc

state_id=ST-P00-A01; solution_ref=SOL-ST-P00-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Rút camera trước startup.

Camera bị app khác chiếm.

Feed trả cùng một frame lặp lại.

PASS nếu không có autonomous motion và recovery chỉ resume sau full revalidation.

### 10. Module ownership / implementation hooks

CameraHealthMonitor

CameraMonitor

SafetyManager

GeometryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P00-A02 — Board geometry/orientation invalid

| State source | ST-P00-A02 — Board geometry/orientation invalid |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Không xác định đáng tin 4 góc/orientation/homography lúc khởi động. |
| Source evidence | Geometry invalid hoặc orientation ambiguous. |
| Requirements | R4,R5,R11,R12 |
| Scenario | S13 |

### 1. Possible root causes

Không nhìn đủ 4 góc/board keypoints.

Corner order/orientation bị đảo.

Camera/board đã dịch từ lần calibration trước.

Homography file/config không khớp camera hiện tại.

### 2. Evidence cần để phân biệt root cause

4 corner/keypoint + confidence.

Reprojection residual/heatmap.

orientation result và geometry_version.

Camera resolution/zoom/config hash.

### 3. Detection / decision logic

Logic đề xuất: Reject geometry nếu corner confidence thấp, orientation mơ hồ, reprojection vượt threshold hoặc config version không khớp.

### 4. Immediate safety / state action

Block PICK/PLACE và mọi board->robot transform.

Đánh dấu geometry/registration invalid.

### 5. Automatic recovery / operator procedure

Thực hiện board pose detection lại.

Nếu auto pose không đáng tin: yêu cầu manual 4-corner calibration.

Tạo geometry_version mới rồi invalidate/refresh robot registration; sau đó rescan board.

### 6. Retry / abort limit

Policy: Không retry motion. Có thể thử tối đa 2 lần geometry reacquisition; sau đó HUMAN_REQUIRED/CAMERA_INVALID tùy nguyên nhân.

### 7. Verification sau recovery

P95 reprojection trong threshold.

Orientation đúng 100% test logic.

Geometry version và registration version đồng bộ.

ObservedBoard được đọc lại dưới geometry mới.

### 8. Logging bắt buộc

state_id=ST-P00-A02; solution_ref=SOL-ST-P00-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Che 1 góc; đảo thứ tự corner; đổi resolution; dịch camera.

PASS nếu geometry giả không bao giờ được dùng để tạo robot pose.

### 10. Module ownership / implementation hooks

GeometryManager

RobotRegistration

CameraHealthMonitor

SafetyManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P00-A03 — Initial observed board mismatch

| State source | ST-P00-A03 — Initial observed board mismatch |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | ObservedBoard khác ExpectedBoard hoặc khác setup được yêu cầu. |
| Source evidence | Thiếu/thừa/sai quân ở một hay nhiều intersection. |
| Requirements | R1,R6,R8,R12 |
| Scenario | S20 |

### 1. Possible root causes

Bàn vật lý chưa setup đúng persisted/initial state.

Perception nhầm một hoặc nhiều quân.

Orientation sai.

Persisted state cũ hoặc sai version.

### 2. Evidence cần để phân biệt root cause

ExpectedBoard và ObservedBoard cell-by-cell.

Confidence/visibility của các cell mismatch.

Orientation/geometry validity.

Persisted state_version và event log cuối.

### 3. Detection / decision logic

Logic đề xuất: Tạo discrepancy set; không tự ghi ObservedBoard đè lên ExpectedBoard.

### 4. Immediate safety / state action

Freeze game start/turn advancement.

Không cho robot tự sửa bàn khi chưa xác định root cause.

### 5. Automatic recovery / operator procedure

Rescan nhiều frame.

Nếu mismatch chỉ do low-confidence: resolve bằng R12.

Nếu physical board ổn định nhưng khác expected: hiển thị chính xác cell mismatch và yêu cầu người sửa hoặc chọn explicit reconcile policy.

Chỉ update expected qua coordinator sau xác nhận hợp lệ.

### 6. Retry / abort limit

Policy: Tối đa 2 rescan; physical mismatch bền vững => HUMAN_REQUIRED trừ khi product có workflow restore được kiểm chứng.

### 7. Verification sau recovery

ExpectedBoard == verified ObservedBoard hoặc operator xác nhận một state mới hợp lệ.

No unresolved discrepancy trước P10.

### 8. Logging bắt buộc

state_id=ST-P00-A03; solution_ref=SOL-ST-P00-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Thiếu 1 quân; thừa 1 quân; đổi loại quân; perception low confidence.

PASS nếu hệ thống không silently accept mismatch.

### 10. Module ownership / implementation hooks

BoardRecognizer

StateStore

BoardReconciler

RuleEngine

ConfidencePolicy

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P00-A04 — Persisted action incomplete after restart

| State source | ST-P00-A04 — Persisted action incomplete after restart |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Persisted CurrentAction cho thấy hệ thống từng crash giữa transaction vật lý. |
| Source evidence | phase != completed; board physical có thể ở trạng thái nửa chừng. |
| Requirements | R6,R10,R13 |
| Scenario | S16 |

### 1. Possible root causes

Process crash/kill khi CurrentAction đang PICK/TRANSPORT/PLACE/CAPTURE.

Persist event chưa được ghi trước physical side-effect.

Shutdown mất điện giữa transaction.

### 2. Evidence cần để phân biệt root cause

Persisted CurrentAction {piece,src,dst,phase,attempt}.

Event log trước crash.

ObservedBoard sau restart; loose-object scan nếu cần.

Robot/gripper status nếu còn truy cập được.

### 3. Detection / decision logic

Logic đề xuất: Nếu phase != COMPLETED thì tuyệt đối không giả định nước đi đã thành công.

### 4. Immediate safety / state action

Giữ robot ở safe stopped/disabled khi restart.

Không auto-commit persisted move.

### 5. Automatic recovery / operator procedure

Reconstruct physical state: source/destination/loose piece/captured piece.

So với expected pre-condition và planned post-condition.

Nếu chỉ có một recovery path được xác minh, tạo recovery transaction mới; nếu không, HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: Không auto-resume physical motion chỉ dựa vào persisted phase. Một lần reconcile đầy đủ; ambiguity => human.

### 7. Verification sau recovery

Board và CurrentAction được reconcile.

Không half-commit.

State_version mới được ghi atomically trước khi tiếp tục.

### 8. Logging bắt buộc

state_id=ST-P00-A04; solution_ref=SOL-ST-P00-A04

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Crash sau close gripper; sau lift; giữa transport; sau release trước verify.

PASS nếu restart không bao giờ tự coi action hoàn tất.

### 10. Module ownership / implementation hooks

StateStore

TransactionManager

BoardReconciler

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 6. P10 — Pre-turn Observation & Synchronization

Mục tiêu phase: Xác nhận scene ổn định và logical/physical board đồng bộ trước lượt.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P10-N01 SOL-ST-P10-N01 | NORMAL | Scene stable and board synchronized |
| ST-P10-T01 SOL-ST-P10-T01 | TRANSIENT / AMBIGUOUS | Human hand present / board occluded |
| ST-P10-T02 SOL-ST-P10-T02 | TRANSIENT / AMBIGUOUS | Robot self-occlusion |
| ST-P10-T03 SOL-ST-P10-T03 | TRANSIENT / AMBIGUOUS | Perception uncertain but scene otherwise stable |
| ST-P10-A01 SOL-ST-P10-A01 | ANOMALY | Board moved relative to calibration |
| ST-P10-A02 SOL-ST-P10-A02 | ANOMALY | Camera feed stale/frozen |
| ST-P10-A03 SOL-ST-P10-A03 | ANOMALY | Expected vs observed desynchronization before turn |

## SOL-ST-P10-N01 — Scene stable and board synchronized

| State source | ST-P10-N01 — Scene stable and board synchronized |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Scene không bị che; geometry valid; ObservedBoard khớp ExpectedBoard. |
| Source evidence | No discrepancy; camera fresh. |
| Requirements | R1,R4,R6,R11,R12,R13 |
| Scenario | S01,S02 |

### Normal handling contract

Xác nhận evidence của state: No discrepancy; camera fresh..

Chỉ cho phép transition sang: WAIT_HUMAN / robot plan tùy turn.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P10-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P10-T01 — Human hand present / board occluded

| State source | ST-P10-T01 — Human hand present / board occluded |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Tay người che một phần bàn; chưa được phép kết luận state. |
| Source evidence | Visibility thấp; multiple cells unknown. |
| Requirements | R1,R7,R11,R12 |
| Scenario | S04,S19 |

### 1. Possible root causes

Người đang đi quân hoặc tay chưa rời vùng bàn.

Tay chỉ chạm/điều chỉnh quân.

Vật thể khác che camera.

### 2. Evidence cần để phân biệt root cause

Hand/person/occlusion signal.

Visibility mask/UNKNOWN cells.

Scene-motion score theo thời gian.

### 3. Detection / decision logic

Logic đề xuất: Scene chưa stable thì không tạo transition từ board snapshot.

### 4. Immediate safety / state action

Robot motion inhibited.

Không update ExpectedBoard/ObservedBoard chính thức từ frame bị che.

### 5. Automatic recovery / operator procedure

Chờ hand-clear + debounce/stability window.

Sau khi clear lấy fresh burst và quay lại state trước.

### 6. Retry / abort limit

Policy: Không tính là failure trong lúc người vẫn tương tác; có timeout hợp lý, sau timeout route ST-P20-A06.

### 7. Verification sau recovery

Nhiều frame liên tiếp không có hand/occlusion.

Board state ổn định trước khi phân tích move.

### 8. Logging bắt buộc

state_id=ST-P10-T01; solution_ref=SOL-ST-P10-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Tay che 1-5 quân 1-5s; chạm quân rồi trả lại.

PASS nếu không tạo fake disappear/move.

### 10. Module ownership / implementation hooks

SceneStabilityGate

SafetyManager

BoardRecognizer

TemporalReasoner

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P10-T02 — Robot self-occlusion

| State source | ST-P10-T02 — Robot self-occlusion |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Cánh tay/gripper che camera trong chính hành động robot. |
| Source evidence | Một vùng bàn bị occluded bởi robot. |
| Requirements | R7,R11,R13 |
| Scenario | S19 |

### 1. Possible root causes

Robot/gripper đi qua line of sight của overhead camera.

Piece đang được mang nằm sau gripper.

### 2. Evidence cần để phân biệt root cause

robot_motion_state.

Robot mask/occlusion region nếu có.

Timestamp interval robot enters/leaves camera ROI.

### 3. Detection / decision logic

Logic đề xuất: Khi robot_motion_state=true và expected self-occlusion, perception output trong vùng che không được commit thành board delta.

### 4. Immediate safety / state action

Giữ CurrentAction theo workflow; suppress semantic commit trong occlusion interval.

### 5. Automatic recovery / operator procedure

Chờ ROBOT_CLEAR checkpoint rồi lấy fresh snapshot/burst.

Verify phase post-condition khi scene clear.

### 6. Retry / abort limit

Policy: Không retry chỉ vì mất nhìn trong expected occlusion. Nếu scene vẫn không observable sau robot clear -> ST-P90-T01/A06.

### 7. Verification sau recovery

Camera clear; post-action snapshot fresh.

ObservedBoard phù hợp expected phase.

### 8. Logging bắt buộc

state_id=ST-P10-T02; solution_ref=SOL-ST-P10-T02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Robot che source/destination trong transport/place.

PASS nếu không sinh fake human move hoặc mất quân.

### 10. Module ownership / implementation hooks

RobotStateAdapter

SceneStabilityGate

BoardRecognizer

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P10-T03 — Perception uncertain but scene otherwise stable

| State source | ST-P10-T03 — Perception uncertain but scene otherwise stable |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Scene ổn định nhưng một/một số cell có confidence thấp hoặc nhiều giả thuyết. |
| Source evidence | UNKNOWN / low confidence. |
| Requirements | R1,R12,R13 |
| Scenario | S14 |

### 1. Possible root causes

Ánh sáng/bóng phản xạ.

Classifier phân vân quân có ký tự tương tự.

Blur nhẹ hoặc piece bị lệch.

Một nguồn sensor/model bất đồng.

### 2. Evidence cần để phân biệt root cause

Per-cell confidence/UNKNOWN.

Temporal votes trên burst.

Geometry residual.

Rule/history consistency.

### 3. Detection / decision logic

Logic đề xuất: Nếu confidence dưới threshold hoặc các nguồn không đồng thuận, output phải là UNKNOWN/AMBIGUOUS chứ không ép nearest class.

### 4. Immediate safety / state action

Không commit move; không motion nếu cell uncertain liên quan action.

### 5. Automatic recovery / operator procedure

Rescan burst với exposure ổn định.

Temporal consensus; dùng ExpectedBoard/rule như evidence nhưng không ép accept.

Nếu vẫn low confidence, escalate HUMAN_REQUIRED hoặc camera/perception issue.

### 6. Retry / abort limit

Policy: Tối đa 2 rescan; không hạ threshold chỉ để pass runtime.

### 7. Verification sau recovery

Confidence/evidence vượt policy và state nhất quán.

Decision path/log chỉ rõ vì sao ALLOW.

### 8. Logging bắt buộc

state_id=ST-P10-T03; solution_ref=SOL-ST-P10-T03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Low-light; glare; piece partly occluded; two models disagree.

PASS nếu uncertain case không bị silent accept.

### 10. Module ownership / implementation hooks

BoardRecognizer

ConfidencePolicy

TemporalReasoner

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P10-A01 — Board moved relative to calibration

| State source | ST-P10-A01 — Board moved relative to calibration |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Bàn dịch/xoay vượt tolerance so với transform đang dùng. |
| Source evidence | Board-motion event / residual tăng / corner shift. |
| Requirements | R4,R5,R11,R13 |
| Scenario | S13 |

### 1. Possible root causes

Bàn bị người đẩy/xoay.

Camera mount rung làm apparent board pose đổi.

Calibration file cũ.

Board keypoint detection drift.

### 2. Evidence cần để phân biệt root cause

Corner/keypoint delta.

Homography/reprojection residual.

geometry_version và robot registration version.

Before/after board image.

### 3. Detection / decision logic

Logic đề xuất: Nếu board pose delta vượt tolerance hoặc residual vượt limit thì geometry invalid ngay.

### 4. Immediate safety / state action

Block robot motion.

Không dùng mapping cũ dù semantic board vẫn đọc được.

### 5. Automatic recovery / operator procedure

Reacquire board pose.

Update/recompute homography.

Recalibrate/refresh board->robot registration theo thiết kế được hỗ trợ.

Rescan full board và reconcile Expected/Observed trước resume.

### 6. Retry / abort limit

Policy: Không auto retry PICK/PLACE trước khi mapping mới VERIFIED.

### 7. Verification sau recovery

R4 geometry PASS + R5 registration valid.

Board sync PASS.

Version chain camera/geometry/registration khớp.

### 8. Logging bắt buộc

state_id=ST-P10-A01; solution_ref=SOL-ST-P10-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Dịch bàn 2/5/10/20 mm; xoay 0.5/1/3°.

PASS nếu mapping cũ bị reject 100% khi vượt threshold.

### 10. Module ownership / implementation hooks

GeometryManager

RobotRegistration

SafetyManager

BoardReconciler

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P10-A02 — Camera feed stale/frozen

| State source | ST-P10-A02 — Camera feed stale/frozen |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Frame lặp/stale khiến hệ thống tưởng scene không đổi. |
| Source evidence | timestamp/frame_id freshness bất thường. |
| Requirements | R11,R12,R13 |
| Scenario | S14 |

### 1. Possible root causes

Camera driver treo.

Buffer không cập nhật.

Capture thread chết nhưng UI còn frame cũ.

Network/USB camera mất kết nối.

### 2. Evidence cần để phân biệt root cause

Monotonic frame_id/timestamp.

Image hash/change statistics.

Capture thread heartbeat.

Device error code.

### 3. Detection / decision logic

Logic đề xuất: Feed được coi stale nếu frame age/heartbeat vượt threshold hoặc frame_id không tiến triển.

### 4. Immediate safety / state action

Immediately inhibit autonomous motion.

Mark CAMERA_INVALID candidate.

### 5. Automatic recovery / operator procedure

Restart capture pipeline/reopen device một lần.

Flush stale buffer.

Require fresh frames -> geometry revalidate -> board rescan.

### 6. Retry / abort limit

Policy: 1 automatic camera restart; lặp lại => ST-P110-X02.

### 7. Verification sau recovery

Fresh frame age trong threshold.

Geometry valid.

ObservedBoard trusted.

### 8. Logging bắt buộc

state_id=ST-P10-A02; solution_ref=SOL-ST-P10-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Freeze capture thread; unplug camera; stale buffer replay.

PASS nếu no-motion và không verify bằng frame cũ.

### 10. Module ownership / implementation hooks

CameraHealthMonitor

CameraMonitor

SafetyManager

GeometryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P10-A03 — Expected vs observed desynchronization before turn

| State source | ST-P10-A03 — Expected vs observed desynchronization before turn |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Trước lượt mới, physical board không còn khớp logical state. |
| Source evidence | Discrepancy set non-empty. |
| Requirements | R1,R6,R12,R13 |
| Scenario | S17,S20 |

### 1. Possible root causes

Manual board modification giữa lượt.

Previous robot/human action chưa verify đúng.

Perception error.

State persistence/server sync lỗi.

### 2. Evidence cần để phân biệt root cause

Full discrepancy set.

Last committed board/state_version.

Last CurrentAction/event log.

Per-cell confidence và geometry validity.

### 3. Detection / decision logic

Logic đề xuất: Desync chỉ được xác nhận sau stable rescan và geometry valid.

### 4. Immediate safety / state action

Freeze next turn; no move planning.

### 5. Automatic recovery / operator procedure

Rescan -> classify discrepancy.

Nếu discrepancy khớp một known uncommitted action: route recovery tương ứng.

Nếu physical modification không giải thích được: HUMAN_REQUIRED.

Không overwrite ExpectedBoard tự động.

### 6. Retry / abort limit

Policy: 2 rescan max; persistent unexplained mismatch => human.

### 7. Verification sau recovery

Một canonical board state được xác nhận.

Expected == Observed.

No incomplete action.

### 8. Logging bắt buộc

state_id=ST-P10-A03; solution_ref=SOL-ST-P10-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Move one piece manually between turns; inject stale server state.

PASS nếu next turn không bắt đầu khi discrepancy còn tồn tại.

### 10. Module ownership / implementation hooks

BoardReconciler

StateStore

BoardRecognizer

ConfidencePolicy

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 7. P20 — Human Turn / Physical Interaction

Mục tiêu phase: Không kết luận move cho tới khi human interaction kết thúc và scene stable.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P20-N01 SOL-ST-P20-N01 | NORMAL | Human thinking / no physical change |
| ST-P20-T01 SOL-ST-P20-T01 | TRANSIENT / AMBIGUOUS | Human touches piece then returns it |
| ST-P20-T02 SOL-ST-P20-T02 | TRANSIENT / AMBIGUOUS | Human lifts piece but move not finished |
| ST-P20-N02 SOL-ST-P20-N02 | NORMAL | Human normal move completed |
| ST-P20-N03 SOL-ST-P20-N03 | NORMAL | Human capture completed |
| ST-P20-A01 SOL-ST-P20-A01 | ANOMALY | Human illegal move |
| ST-P20-A02 SOL-ST-P20-A02 | ANOMALY | Two or more pieces changed |
| ST-P20-A03 SOL-ST-P20-A03 | ANOMALY | Pieces swapped / replaced |
| ST-P20-A04 SOL-ST-P20-A04 | ANOMALY | Piece added or removed without legal move |
| ST-P20-A05 SOL-ST-P20-A05 | ANOMALY | Human places piece off-grid |
| ST-P20-A06 SOL-ST-P20-A06 | ANOMALY | Occlusion never resolves / move cannot be observed |

## SOL-ST-P20-N01 — Human thinking / no physical change

| State source | ST-P20-N01 — Human thinking / no physical change |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Người chơi suy nghĩ; board không đổi. |
| Source evidence | Stable board equals previous state. |
| Requirements | R6,R7,R13 |
| Scenario | S01 |

### Normal handling contract

Xác nhận evidence của state: Stable board equals previous state..

Chỉ cho phép transition sang: Tiếp tục chờ.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P20-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-T01 — Human touches piece then returns it

| State source | ST-P20-T01 — Human touches piece then returns it |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Quân bị chạm/nhấc nhẹ nhưng cuối cùng trở về vị trí cũ. |
| Source evidence | Transient disappearance rồi same-state restore. |
| Requirements | R7,R11,R12 |
| Scenario | S04 |

### 1. Possible root causes

Người nhấc nhẹ rồi đặt lại.

Detection transient khi tay che quân.

Quân rung nhưng logical position không đổi.

### 2. Evidence cần để phân biệt root cause

Temporal board sequence trước/trong/sau.

Hand/occlusion signal.

Final stable board.

### 3. Detection / decision logic

Logic đề xuất: Không emit MOVE nếu final stable state == pre-turn state.

### 4. Immediate safety / state action

Giữ WAIT_HUMAN; không commit.

### 5. Automatic recovery / operator procedure

Chờ scene stable; compare final state với baseline.

Nếu same-state -> NO_CHANGE và tiếp tục chờ lượt.

### 6. Retry / abort limit

Policy: Không retry; đây là transient hợp lệ.

### 7. Verification sau recovery

Final ObservedBoard == pre-turn ExpectedBoard.

No move event emitted.

### 8. Logging bắt buộc

state_id=ST-P20-T01; solution_ref=SOL-ST-P20-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Touch/rotate/lift 1-2s rồi trả đúng ô.

PASS false-move rate theo R7 target.

### 10. Module ownership / implementation hooks

TemporalReasoner

SceneStabilityGate

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-T02 — Human lifts piece but move not finished

| State source | ST-P20-T02 — Human lifts piece but move not finished |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Source tạm trống nhưng destination chưa ổn định; đây chưa phải nước đi hoàn chỉnh. |
| Source evidence | Piece absent/hand present/scene unstable. |
| Requirements | R7,R11,R12,R13 |
| Scenario | Human delayed placement |

### 1. Possible root causes

Người đang giữ quân giữa source và destination.

Delayed placement.

Hand che destination.

### 2. Evidence cần để phân biệt root cause

Source temporarily empty.

Hand present/scene unstable.

No stable destination yet.

### 3. Detection / decision logic

Logic đề xuất: Source disappearance trong khi hand present không đủ để commit một move.

### 4. Immediate safety / state action

Không phân tích luật/không chuyển turn.

### 5. Automatic recovery / operator procedure

Chờ hand clear + final stable board.

Nếu timeout/không quan sát được -> ST-P20-A06.

### 6. Retry / abort limit

Policy: Không giới hạn thời gian suy nghĩ; chỉ timeout riêng cho 'hand interaction active' theo UX/safety.

### 7. Verification sau recovery

Stable final state có unique move/capture hoặc returns to NO_CHANGE.

### 8. Logging bắt buộc

state_id=ST-P20-T02; solution_ref=SOL-ST-P20-T02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Giữ quân trên tay vài giây; đặt chậm.

PASS nếu không prematurely commit.

### 10. Module ownership / implementation hooks

SceneStabilityGate

TemporalReasoner

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-N02 — Human normal move completed

| State source | ST-P20-N02 — Human normal move completed |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Một quân rời source và xuất hiện ở destination trống. |
| Source evidence | Một disappeared + một appeared phù hợp. |
| Requirements | R1,R6,R7 |
| Scenario | S01 |

### Normal handling contract

Xác nhận evidence của state: Một disappeared + một appeared phù hợp..

Chỉ cho phép transition sang: P30 interpretation.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P20-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-N03 — Human capture completed

| State source | ST-P20-N03 — Human capture completed |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Quân người chơi đi tới vị trí có quân đối phương và trạng thái cuối phản ánh capture. |
| Source evidence | Semantic board thay đổi đúng mẫu capture. |
| Requirements | R1,R6,R7 |
| Scenario | S02 |

### Normal handling contract

Xác nhận evidence của state: Semantic board thay đổi đúng mẫu capture..

Chỉ cho phép transition sang: P30 interpretation.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P20-N03.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-A01 — Human illegal move

| State source | ST-P20-A01 — Human illegal move |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Observed transition tương ứng một nước đi trái luật hoặc sai lượt. |
| Source evidence | Candidate transition tồn tại nhưng R8 reject. |
| Requirements | R1,R7,R8,R12 |
| Scenario | S05 |

### 1. Possible root causes

Người đi sai luật.

Sai lượt.

Perception candidate sai dù physical move thật có thể hợp lệ.

ExpectedBoard đã desync.

### 2. Evidence cần để phân biệt root cause

Stable rescan.

Candidate move + R8 reject reason.

ExpectedBoard/turn.

Confidence of involved cells.

### 3. Detection / decision logic

Logic đề xuất: Chỉ gọi 'illegal human move' sau khi scene stable, perception đủ chắc và rule engine reject.

### 4. Immediate safety / state action

Không commit; không plan robot response.

### 5. Automatic recovery / operator procedure

Rescan một lần để loại perception error.

Nếu vẫn illegal: hiển thị src/dst/reason và yêu cầu người trả board về ExpectedBoard hoặc thực hiện nước hợp lệ.

Sau correction phải resync trước khi chờ move mới.

### 6. Retry / abort limit

Policy: 1 confirmation rescan; sau đó human correction, không auto 'sửa luật' bằng robot mặc định.

### 7. Verification sau recovery

ObservedBoard trở lại ExpectedBoard hoặc một legal transition mới được nhận.

No illegal move in event log as committed.

### 8. Logging bắt buộc

state_id=ST-P20-A01; solution_ref=SOL-ST-P20-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Sai luật từng quân; wrong turn; general-facing edge case.

PASS illegal acceptance = 0.

### 10. Module ownership / implementation hooks

RuleEngine

TemporalReasoner

BoardReconciler

UI

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-A02 — Two or more pieces changed

| State source | ST-P20-A02 — Two or more pieces changed |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Có nhiều thay đổi độc lập trong cùng lượt. |
| Source evidence | >1 source/destination pair hoặc nhiều cell đổi. |
| Requirements | R1,R6,R7,R12 |
| Scenario | S06,S17 |

### 1. Possible root causes

Người vô tình/chủ ý di chuyển >1 quân.

Một quân bị đẩy collateral.

Occlusion khiến perception tạo nhiều delta giả.

### 2. Evidence cần để phân biệt root cause

Stable full-board rescan.

Discrepancy set/candidate list.

Hand-clear signal; confidence per changed cell.

### 3. Detection / decision logic

Logic đề xuất: Sau stable rescan, nếu >1 independent change vẫn tồn tại thì ANOMALY; không chọn random một pair.

### 4. Immediate safety / state action

Freeze turn; no commit.

### 5. Automatic recovery / operator procedure

Rescan burst.

Nếu một delta biến mất do perception -> re-evaluate.

Nếu nhiều physical changes bền vững: yêu cầu người restore ExpectedBoard; cung cấp danh sách cell cần sửa.

### 6. Retry / abort limit

Policy: 2 rescan max; persistent multi-change => HUMAN_REQUIRED.

### 7. Verification sau recovery

ExpectedBoard restored hoặc unique legal transition + no collateral change.

Discrepancy set empty.

### 8. Logging bắt buộc

state_id=ST-P20-A02; solution_ref=SOL-ST-P20-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Move two pieces; push neighbor; occlusion-generated fake deltas.

PASS nếu multiple hypotheses được gắn AMBIGUOUS/ANOMALY.

### 10. Module ownership / implementation hooks

BoardReconciler

TemporalReasoner

ConfidencePolicy

UI

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-A03 — Pieces swapped / replaced

| State source | ST-P20-A03 — Pieces swapped / replaced |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Hai quân bị tráo vị trí hoặc một quân bị thay bằng loại khác không theo move hợp lệ. |
| Source evidence | Identity mismatch không giải thích được bằng một legal transition. |
| Requirements | R1,R6,R7,R8 |
| Scenario | Cheating/error family |

### 1. Possible root causes

Hai quân bị tráo vị trí.

Một quân bị thay bằng quân khác.

Classifier identity error.

### 2. Evidence cần để phân biệt root cause

Semantic identity at changed cells across burst.

Physical piece count.

ExpectedBoard identity.

Optional manual close-up/secondary view.

### 3. Detection / decision logic

Logic đề xuất: Identity mismatch phải được xác nhận qua multiple frames; rule engine không được 'bẻ' observation thành move hợp lệ.

### 4. Immediate safety / state action

No commit; freeze turn.

### 5. Automatic recovery / operator procedure

Rescan.

Nếu identity uncertainty -> perception escalation.

Nếu swap/replace thật: yêu cầu human restore exact expected identities; then verify full board.

### 6. Retry / abort limit

Policy: Không auto-rearrange nhiều identity bằng robot ở baseline; human required.

### 7. Verification sau recovery

All affected cells match ExpectedBoard after correction.

No unresolved identity mismatch.

### 8. Logging bắt buộc

state_id=ST-P20-A03; solution_ref=SOL-ST-P20-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Swap two same-color pieces; replace one piece class; induce class confusion.

PASS nếu system distinguishes perception uncertainty vs persistent physical mismatch.

### 10. Module ownership / implementation hooks

BoardRecognizer

BoardReconciler

RuleEngine

UI

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-A04 — Piece added or removed without legal move

| State source | ST-P20-A04 — Piece added or removed without legal move |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Một quân biến mất hoặc xuất hiện không có transition hợp lệ. |
| Source evidence | Count/identity mismatch. |
| Requirements | R1,R6,R7,R8 |
| Scenario | Cheating/error family |

### 1. Possible root causes

Người lấy quân khỏi bàn không theo capture.

Thêm quân dự phòng lên bàn.

Piece rolled/fell out of view.

False positive/negative perception.

### 2. Evidence cần để phân biệt root cause

Piece count + semantic board across stable frames.

Loose-piece detector/recovery ROI.

ExpectedBoard.

### 3. Detection / decision logic

Logic đề xuất: Persistent count/identity mismatch not explainable by one legal transition => anomaly.

### 4. Immediate safety / state action

No commit; search/reconcile.

### 5. Automatic recovery / operator procedure

Rescan full scene including recovery ROI.

Nếu missing piece được tìm off-grid -> classify loose piece.

Nếu physical add/remove confirmed -> human restores ExpectedBoard.

### 6. Retry / abort limit

Policy: 2 rescan; unknown missing piece => PIECE_LOST/HUMAN_REQUIRED.

### 7. Verification sau recovery

Count and identities reconcile with ExpectedBoard.

No unknown loose object.

### 8. Logging bắt buộc

state_id=ST-P20-A04; solution_ref=SOL-ST-P20-A04

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Remove one piece; add spare piece; place piece just outside board.

PASS if system doesn't silently normalize count.

### 10. Module ownership / implementation hooks

BoardRecognizer

LoosePieceDetector

BoardReconciler

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-A05 — Human places piece off-grid

| State source | ST-P20-A05 — Human places piece off-grid |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Quân được đặt giữa giao điểm hoặc ngoài vị trí logical hợp lệ. |
| Source evidence | Loose/off-grid object; board semantic mismatch. |
| Requirements | R2,R3,R6,R7 |
| Scenario | Off-grid human error |

### 1. Possible root causes

Người đặt quân giữa hai giao điểm.

Destination estimate near boundary.

Board recognizer không gán được logical cell.

### 2. Evidence cần để phân biệt root cause

Loose object center/nearest intersection distance.

Candidate move context src/piece.

Physical offset in mm/cell.

### 3. Detection / decision logic

Logic đề xuất: Nếu distance-to-intersection > on-grid tolerance thì state off-grid; không snap/clamp thành ô gần nhất.

### 4. Immediate safety / state action

Freeze turn; no commit.

### 5. Automatic recovery / operator procedure

Nếu human interaction còn active: yêu cầu người đặt lại đúng giao điểm.

Nếu người đã rời: UI highlight nearest intended cell/candidate; baseline policy = human correction.

Auto correction bằng robot chỉ khi sau này được validate riêng.

### 6. Retry / abort limit

Policy: Không auto retry robotic correction trong baseline.

### 7. Verification sau recovery

Piece center nằm trong on-grid tolerance và legal move can be uniquely validated.

Board synchronized.

### 8. Logging bắt buộc

state_id=ST-P20-A05; solution_ref=SOL-ST-P20-A05

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Place at offsets beyond threshold in 8 directions.

PASS nếu no false snap to grid.

### 10. Module ownership / implementation hooks

LoosePieceDetector

PhysicalLocalizer

TemporalReasoner

UI

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P20-A06 — Occlusion never resolves / move cannot be observed

| State source | ST-P20-A06 — Occlusion never resolves / move cannot be observed |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Tay/vật cản che lâu hoặc camera không có snapshot sạch để kết luận move. |
| Source evidence | No stable observable state. |
| Requirements | R7,R11,R12,R13 |
| Scenario | S14/S15 related |

### 1. Possible root causes

Tay/người/vật cản che bàn quá lâu.

Camera angle tạo permanent blind spot.

Camera blur/failure sau human action.

### 2. Evidence cần để phân biệt root cause

Occlusion duration.

Visibility mask.

Camera health/freshness.

### 3. Detection / decision logic

Logic đề xuất: Nếu không đạt scene-stable + visibility requirement trước timeout thì move outcome = UNKNOWN.

### 4. Immediate safety / state action

No commit; inhibit robot response.

### 5. Automatic recovery / operator procedure

Yêu cầu người rời vùng camera/clear obstacle.

Rescan khi clear.

Nếu camera không phục hồi -> CAMERA_INVALID; nếu trạng thái vẫn không biết -> HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: Một recovery wait window + 2 rescan; không đoán move từ partial evidence.

### 7. Verification sau recovery

Full required ROI visible and stable; unique observed state.

### 8. Logging bắt buộc

state_id=ST-P20-A06; solution_ref=SOL-ST-P20-A06

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Hand stays over board beyond timeout; cloth/object covers cells.

PASS if move not committed from obscured frames.

### 10. Module ownership / implementation hooks

SceneStabilityGate

CameraHealthMonitor

ConfidencePolicy

UI

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 8. P30 — Move Interpretation & Rule Validation

Mục tiêu phase: Suy ra transition, dùng rule engine để validate nhưng không dùng luật để che giấu perception uncertainty.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P30-N01 SOL-ST-P30-N01 | NORMAL | Unique transition identified |
| ST-P30-N02 SOL-ST-P30-N02 | NORMAL | Transition legal and consistent |
| ST-P30-T01 SOL-ST-P30-T01 | TRANSIENT / AMBIGUOUS | Multiple transition hypotheses |
| ST-P30-T02 SOL-ST-P30-T02 | TRANSIENT / AMBIGUOUS | Legal move but perception confidence insufficient |
| ST-P30-A01 SOL-ST-P30-A01 | ANOMALY | Observed transition remains illegal after stable rescan |
| ST-P30-A02 SOL-ST-P30-A02 | ANOMALY | ExpectedBoard and observed transition conflict |
| ST-P30-A03 SOL-ST-P30-A03 | ANOMALY | Board orientation interpretation inconsistent |

## SOL-ST-P30-N01 — Unique transition identified

| State source | ST-P30-N01 — Unique transition identified |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Từ before/after state suy ra duy nhất src,dst,piece/change_type. |
| Source evidence | One high-confidence transition candidate. |
| Requirements | R6,R7,R12 |
| Scenario | S01,S02 |

### Normal handling contract

Xác nhận evidence của state: One high-confidence transition candidate..

Chỉ cho phép transition sang: Rule validation.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P30-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P30-N02 — Transition legal and consistent

| State source | ST-P30-N02 — Transition legal and consistent |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Rule engine xác nhận đúng luật, đúng lượt và consistent với ExpectedBoard. |
| Source evidence | LEGAL with reason; no discrepancy. |
| Requirements | R6,R8,R13 |
| Scenario | S01,S02 |

### Normal handling contract

Xác nhận evidence của state: LEGAL with reason; no discrepancy..

Chỉ cho phép transition sang: Commit human move / plan robot response.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P30-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P30-T01 — Multiple transition hypotheses

| State source | ST-P30-T01 — Multiple transition hypotheses |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Có từ hai cách giải thích hợp lý trở lên cho cùng observation. |
| Source evidence | Candidate list size > 1. |
| Requirements | R7,R8,R12 |
| Scenario | S17 |

### 1. Possible root causes

Perception noise tạo nhiều src/dst candidate.

Hai cell low confidence.

Nhiều physical changes thật.

Capture ambiguity.

### 2. Evidence cần để phân biệt root cause

Candidate list with scores.

Stable before/after board.

Rule validity for each candidate.

Per-cell confidence.

### 3. Detection / decision logic

Logic đề xuất: Không chọn candidate chỉ vì score cao nhất nếu chênh lệch/evidence chưa đủ.

### 4. Immediate safety / state action

No commit; mark AMBIGUOUS.

### 5. Automatic recovery / operator procedure

Rescan fresh burst.

Dùng rule constraints để loại impossible candidate, nhưng không ép accept nếu perception mơ hồ.

Nếu còn >1 plausible hypothesis => HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: 2 rescan; after that no autonomous resolution.

### 7. Verification sau recovery

Exactly one candidate remains and perception confidence passes policy.

Candidate legal and consistent with ExpectedBoard.

### 8. Logging bắt buộc

state_id=ST-P30-T01; solution_ref=SOL-ST-P30-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Construct two plausible transitions; noisy capture.

PASS ambiguity recall >= target and random choice = 0.

### 10. Module ownership / implementation hooks

TemporalReasoner

RuleEngine

ConfidencePolicy

BoardReconciler

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P30-T02 — Legal move but perception confidence insufficient

| State source | ST-P30-T02 — Legal move but perception confidence insufficient |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Rule engine cho phép candidate nhưng perception evidence chưa đủ chắc. |
| Source evidence | low confidence / unstable fusion. |
| Requirements | R1,R7,R8,R12 |
| Scenario | S14 |

### 1. Possible root causes

Candidate được rule engine chấp nhận nhưng image evidence yếu.

Model overconfident/underconfident.

History makes a move plausible but not visually proven.

### 2. Evidence cần để phân biệt root cause

Raw model confidence.

Temporal consensus.

Geometry quality.

Rule result separated from perception score.

### 3. Detection / decision logic

Logic đề xuất: LEGAL không được nâng một low-confidence perception thành confirmed move.

### 4. Immediate safety / state action

No commit; request more evidence.

### 5. Automatic recovery / operator procedure

Fresh burst/rescan; optional alternate recognizer if available.

If confidence remains low, ask human confirm/correct rather than accept by rule alone.

### 6. Retry / abort limit

Policy: 2 rescan; then HUMAN_REQUIRED/UNKNOWN.

### 7. Verification sau recovery

Perception evidence passes threshold independently enough; rule remains legal.

### 8. Logging bắt buộc

state_id=ST-P30-T02; solution_ref=SOL-ST-P30-T02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Inject legal move with blurred destination; model confident wrong case.

PASS if rule layer never masks perception uncertainty.

### 10. Module ownership / implementation hooks

BoardRecognizer

TemporalReasoner

RuleEngine

ConfidencePolicy

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P30-A01 — Observed transition remains illegal after stable rescan

| State source | ST-P30-A01 — Observed transition remains illegal after stable rescan |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Sau khi scene đã ổn định và quan sát lại, transition vẫn trái luật. |
| Source evidence | R8 rejects same physical state. |
| Requirements | R6,R8,R12 |
| Scenario | S05 |

### 1. Possible root causes

Human truly made illegal move.

Expected turn/rule state wrong.

Persistent perception error.

### 2. Evidence cần để phân biệt root cause

Stable repeated ObservedBoard.

R8 reject reason.

ExpectedBoard/turn version.

Confidence on changed cells.

### 3. Detection / decision logic

Logic đề xuất: After one stable rescan returns same physical state and high confidence, classify persistent illegal.

### 4. Immediate safety / state action

No commit/robot response.

### 5. Automatic recovery / operator procedure

Present exact illegal reason and expected correction.

Human restores board or makes valid move.

If rule state itself suspect, route ST-P30-A02 before blaming human.

### 6. Retry / abort limit

Policy: No autonomous board correction baseline; human correction required.

### 7. Verification sau recovery

Board returns expected state or new legal transition verified.

### 8. Logging bắt buộc

state_id=ST-P30-A01; solution_ref=SOL-ST-P30-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Repeat illegal board state across scans.

PASS illegal state never reaches P100.

### 10. Module ownership / implementation hooks

RuleEngine

BoardReconciler

UI

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P30-A02 — ExpectedBoard and observed transition conflict

| State source | ST-P30-A02 — ExpectedBoard and observed transition conflict |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Transition có thể tự hợp lý nhưng không thể áp lên ExpectedBoard hiện tại. |
| Source evidence | source piece/turn/state mismatch. |
| Requirements | R6,R7,R8 |
| Scenario | S16,S20 |

### 1. Possible root causes

Stale ExpectedBoard.

Previous commit lost/duplicated.

Server/local state conflict.

Wrong orientation or piece identity.

### 2. Evidence cần để phân biệt root cause

ExpectedBoard + version; event log.

ObservedBoard; last committed move.

Server state if used.

Orientation/geometry validity.

### 3. Detection / decision logic

Logic đề xuất: A transition cannot be applied if source piece/turn does not exist in ExpectedBoard; treat as state conflict, not legal move.

### 4. Immediate safety / state action

Freeze game progression and all robot planning.

### 5. Automatic recovery / operator procedure

Rescan physical board.

Replay event log from last known good version.

Compare local/server/physical state.

Choose canonical state only through explicit reconcile policy; otherwise HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: No auto overwrite on first conflict. One deterministic replay/reconcile attempt.

### 7. Verification sau recovery

Single canonical state established; ExpectedBoard == verified physical board.

State_version monotonic.

### 8. Logging bắt buộc

state_id=ST-P30-A02; solution_ref=SOL-ST-P30-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Stale memory; skipped commit; server one move ahead.

PASS if conflict is surfaced rather than silently corrected.

### 10. Module ownership / implementation hooks

StateStore

BoardReconciler

RuleEngine

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P30-A03 — Board orientation interpretation inconsistent

| State source | ST-P30-A03 — Board orientation interpretation inconsistent |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Một move chỉ “hợp lệ” nếu board orientation bị hiểu ngược. |
| Source evidence | coordinate/side mapping conflict. |
| Requirements | R4,R8,R12 |
| Scenario | Orientation failure |

### 1. Possible root causes

Board detected flipped 180°.

Corner ordering reversed.

Player-side mapping/config wrong.

### 2. Evidence cần để phân biệt root cause

Orientation classifier/keypoints.

Known initial/expected piece-side distribution.

Homography corner order.

Rule validity under both orientations only as diagnostic.

### 3. Detection / decision logic

Logic đề xuất: Orientation must be validated independently; do not choose orientation because it makes move legal.

### 4. Immediate safety / state action

Invalidate geometry/state mapping; no motion/commit.

### 5. Automatic recovery / operator procedure

Re-detect corners/orientation.

Require explicit known orientation marker or manual confirmation if ambiguous.

Rebuild transform and re-read full board.

### 6. Retry / abort limit

Policy: No rule-based auto-flip. 2 geometry attempts then human.

### 7. Verification sau recovery

Orientation 100% consistent with configured side.

Reprojection valid and full board sync.

### 8. Logging bắt buộc

state_id=ST-P30-A03; solution_ref=SOL-ST-P30-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Rotate board 180°; swap corner order.

PASS if system blocks instead of silently flipping coordinates.

### 10. Module ownership / implementation hooks

GeometryManager

BoardRecognizer

RuleEngine

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 9. P40 — Robot Planning & Pre-motion Gate

Mục tiêu phase: Mọi robot move phải qua geometry, registration, reachability, confidence và safety gate.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P40-N01 SOL-ST-P40-N01 | NORMAL | Normal robot move planned |
| ST-P40-N02 SOL-ST-P40-N02 | NORMAL | Capture move planned |
| ST-P40-T01 SOL-ST-P40-T01 | TRANSIENT / AMBIGUOUS | Target localization available but uncertainty near limit |
| ST-P40-A01 SOL-ST-P40-A01 | ANOMALY | Target outside robot workspace / auto-recovery zone |
| ST-P40-A02 SOL-ST-P40-A02 | ANOMALY | Board-to-robot mapping invalid |
| ST-P40-A03 SOL-ST-P40-A03 | ANOMALY | Scene unsafe or human present before motion |

## SOL-ST-P40-N01 — Normal robot move planned

| State source | ST-P40-N01 — Normal robot move planned |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Move robot không capture được lập kế hoạch với src,dst và target pose hợp lệ. |
| Source evidence | Mapping valid; workspace reachable; scene safe. |
| Requirements | R5,R8,R9,R11,R13 |
| Scenario | Normal robot move |

### Normal handling contract

Xác nhận evidence của state: Mapping valid; workspace reachable; scene safe..

Chỉ cho phép transition sang: P60 PICK.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P40-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P40-N02 — Capture move planned

| State source | ST-P40-N02 — Capture move planned |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Move robot là capture; cần remove quân đối phương trước khi move quân của robot. |
| Source evidence | CurrentAction includes capture sub-transaction. |
| Requirements | R6,R8,R9,R13 |
| Scenario | S02 |

### Normal handling contract

Xác nhận evidence của state: CurrentAction includes capture sub-transaction..

Chỉ cho phép transition sang: P50 CAPTURE REMOVAL.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P40-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P40-T01 — Target localization available but uncertainty near limit

| State source | ST-P40-T01 — Target localization available but uncertainty near limit |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Có target vật lý nhưng uncertainty gần tolerance, chưa nên motion ngay. |
| Source evidence | uncertainty high / near obstacle. |
| Requirements | R3,R12,R13 |
| Scenario | S03 |

### 1. Possible root causes

Piece localization variance high.

Piece near neighbor/obstacle.

Perspective/segmentation uncertainty near board edge.

### 2. Evidence cần để phân biệt root cause

Target XY + covariance/uncertainty.

Available grasp margin/tolerance budget.

Distance to neighboring pieces/obstacles.

### 3. Detection / decision logic

Logic đề xuất: ALLOW only if uncertainty bound + registration error + safety reserve fits available grasp margin and clearance.

### 4. Immediate safety / state action

Hold before motion.

### 5. Automatic recovery / operator procedure

Take fresh high-quality snapshot/localize again.

Prefer a better localization backend if available.

If uncertainty remains near/over limit => no motion/HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: 2 localization attempts; no threshold relaxation at runtime.

### 7. Verification sau recovery

Final target uncertainty within derived tolerance budget.

Clearance check passes.

### 8. Logging bắt buộc

state_id=ST-P40-T01; solution_ref=SOL-ST-P40-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Offsets near tolerance; edge cells; neighbor close.

PASS if uncertain target never starts PICK.

### 10. Module ownership / implementation hooks

PhysicalLocalizer

RobotRegistration

ConfidencePolicy

SafetyManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P40-A01 — Target outside robot workspace / auto-recovery zone

| State source | ST-P40-A01 — Target outside robot workspace / auto-recovery zone |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Target pose không thể với tới an toàn. |
| Source evidence | reachability false / zone invalid. |
| Requirements | R5,R9,R11,R12 |
| Scenario | S12 |

### 1. Possible root causes

Target truly outside arm reach.

Bad registration generates impossible pose.

Recovery ROI configured larger than robot workspace.

Joint/orientation constraints make pose unsafe.

### 2. Evidence cần để phân biệt root cause

IK/reachability or SDK preflight result.

Workspace/recovery-zone classification.

Registration validity.

### 3. Detection / decision logic

Logic đề xuất: Reachability must be checked before motion; no clamping target to nearest reachable point.

### 4. Immediate safety / state action

No motion.

### 5. Automatic recovery / operator procedure

If registration invalid route A02.

If target physically outside allowed zone, require human reposition/recovery.

For optional recovery pad, use only pre-taught safe pose, not ad-hoc extrapolation.

### 6. Retry / abort limit

Policy: No autonomous retry to unreachable target.

### 7. Verification sau recovery

Target either becomes reachable after explicit human correction or state terminates HUMAN_REQUIRED.

No robot command issued outside bounds.

### 8. Logging bắt buộc

state_id=ST-P40-A01; solution_ref=SOL-ST-P40-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Place loose piece just outside workspace; corrupt mapping.

PASS if command is rejected before robot moves.

### 10. Module ownership / implementation hooks

RobotRegistration

MotionPlanner

SafetyManager

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P40-A02 — Board-to-robot mapping invalid

| State source | ST-P40-A02 — Board-to-robot mapping invalid |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Geometry version và robot registration không còn hợp lệ. |
| Source evidence | transform invalid/version mismatch. |
| Requirements | R4,R5,R11,R13 |
| Scenario | S13 |

### 1. Possible root causes

Board moved.

Teaching points/TCP changed.

Geometry_version mismatch registration version.

Calibration residual too high.

### 2. Evidence cần để phân biệt root cause

geometry_version vs registration_version.

Registration residual/heatmap.

TCP/tool config hash.

### 3. Detection / decision logic

Logic đề xuất: Any version mismatch or invalid residual invalidates all board->robot poses.

### 4. Immediate safety / state action

Block PICK/PLACE immediately.

### 5. Automatic recovery / operator procedure

Re-run geometry then registration calibration/validation.

Touch/check calibration points as required.

Re-read board before planning again.

### 6. Retry / abort limit

Policy: No motion retry with old mapping.

### 7. Verification sau recovery

Registration benchmark/spot-check passes.

Version chain matches.

Board sync passes.

### 8. Logging bắt buộc

state_id=ST-P40-A02; solution_ref=SOL-ST-P40-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Shift board; change teaching point; change TCP config.

PASS mapping old version rejected 100%.

### 10. Module ownership / implementation hooks

GeometryManager

RobotRegistration

ConfigManager

SafetyManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P40-A03 — Scene unsafe or human present before motion

| State source | ST-P40-A03 — Scene unsafe or human present before motion |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Có tay người/vật cản hoặc scene UNKNOWN trước khi robot chạy. |
| Source evidence | BLOCKED/HUMAN_PRESENT/UNKNOWN. |
| Requirements | R11,R12,R13 |
| Scenario | S15 |

### 1. Possible root causes

Hand/person inside safety zone.

Unexpected obstacle.

Scene/camera health UNKNOWN.

Safety interlock not ready.

### 2. Evidence cần để phân biệt root cause

Safety state.

Human/obstacle detector or native safety signal.

Camera health/geometry validity.

### 3. Detection / decision logic

Logic đề xuất: Pre-motion gate requires SAFE; UNKNOWN is not SAFE.

### 4. Immediate safety / state action

Do not issue motion command.

If hardware safety stop triggered, preserve it.

### 5. Automatic recovery / operator procedure

Wait for scene to clear; require debounce + fresh observation.

If persistent/critical -> ST-P110-X04.

Never bypass native robot safety/E-stop.

### 6. Retry / abort limit

Policy: No automatic retry while safety != SAFE.

### 7. Verification sau recovery

Safety state SAFE continuously for required window.

Camera/geometry valid.

Explicit coordinator transition back to planning.

### 8. Logging bắt buộc

state_id=ST-P40-A03; solution_ref=SOL-ST-P40-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Hand enters zone just before command; obstacle; camera invalid.

PASS unsafe auto-action rate = 0.

### 10. Module ownership / implementation hooks

SafetyManager

SceneStabilityGate

MotionPlanner

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 10. P50 — Capture Removal (Conditional)

Mục tiêu phase: Capture là sub-transaction; captured piece phải được remove + verify trước khi pick quân của robot.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P50-N01 SOL-ST-P50-N01 | NORMAL | Captured piece target localized |
| ST-P50-N02 SOL-ST-P50-N02 | NORMAL | Captured piece removed successfully |
| ST-P50-A01 SOL-ST-P50-A01 | ANOMALY | Missed captured-piece grasp |
| ST-P50-A02 SOL-ST-P50-A02 | ANOMALY | Captured piece dropped during removal |
| ST-P50-A03 SOL-ST-P50-A03 | ANOMALY | Wrong piece removed |
| ST-P50-A04 SOL-ST-P50-A04 | ANOMALY | Captured piece not cleared from board/bin path |

## SOL-ST-P50-N01 — Captured piece target localized

| State source | ST-P50-N01 — Captured piece target localized |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Quân đối phương tại destination được định vị đủ tốt để gắp bỏ. |
| Source evidence | Physical target pose valid. |
| Requirements | R3,R5,R9 |
| Scenario | S02 |

### Normal handling contract

Xác nhận evidence của state: Physical target pose valid..

Chỉ cho phép transition sang: Capture pick.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P50-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P50-N02 — Captured piece removed successfully

| State source | ST-P50-N02 — Captured piece removed successfully |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Quân bị ăn đã rời board và đi tới vùng/bin dự kiến. |
| Source evidence | Destination intersection now free/verified. |
| Requirements | R6,R9,R10,R13 |
| Scenario | S02 |

### Normal handling contract

Xác nhận evidence của state: Destination intersection now free/verified..

Chỉ cho phép transition sang: P60 pick moving piece.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P50-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P50-A01 — Missed captured-piece grasp

| State source | ST-P50-A01 — Missed captured-piece grasp |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Gripper đóng nhưng quân bị ăn vẫn ở destination. |
| Source evidence | Post-pick observation unchanged. |
| Requirements | R9,R10,R13 |
| Scenario | Capture failure |

### 1. Possible root causes

Target localization off.

Gripper opening/height wrong.

Piece shifted before close.

Gripper mechanical slip/failure.

### 2. Evidence cần để phân biệt root cause

Post-pick destination cell still occupied.

Gripper feedback if available.

Pre-pick target XY/uncertainty.

Robot pose/error.

### 3. Detection / decision logic

Logic đề xuất: Command-success alone cannot mark captured piece removed; verify physical disappearance from destination.

### 4. Immediate safety / state action

Stop capture transaction; do not pick moving piece yet.

### 5. Automatic recovery / operator procedure

Relocalize captured piece.

If scene clear and no collateral change, retry capture pick once with corrected XY/Z.

Repeated failure => HUMAN_REQUIRED/ROBOT_ERROR depending evidence.

### 6. Retry / abort limit

Policy: Default physical retry <=1 after re-localization; configurable only after benchmark.

### 7. Verification sau recovery

Captured piece absent from destination and either verified in bin/out area or transaction evidence consistent.

No other cell changed.

### 8. Logging bắt buộc

state_id=ST-P50-A01; solution_ref=SOL-ST-P50-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Intentional miss by offset; gripper no-close.

PASS if P60 never starts after unverified removal.

### 10. Module ownership / implementation hooks

ManipulationController

PostActionVerifier

PhysicalLocalizer

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P50-A02 — Captured piece dropped during removal

| State source | ST-P50-A02 — Captured piece dropped during removal |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Quân bị ăn rơi trước khi vào bin/out area. |
| Source evidence | Loose object appears / carried piece missing. |
| Requirements | R2,R3,R10 |
| Scenario | Drop family |

### 1. Possible root causes

Weak grasp/slip.

Transport acceleration too high.

Gripper opens unexpectedly.

Collision/vibration.

### 2. Evidence cần để phân biệt root cause

Captured piece destination becomes empty.

Loose piece detector location.

Gripper feedback/trajectory phase.

### 3. Detection / decision logic

Logic đề xuất: Classify drop location using same drop taxonomy as P70; do not continue capture transaction until captured piece is cleared.

### 4. Immediate safety / state action

Stop current capture flow at safe pose if possible.

No moving-piece pick.

### 5. Automatic recovery / operator procedure

If loose piece is isolated in recovery zone: localize -> repick -> move to capture bin/out area -> verify.

If overlap/out-of-view => HUMAN_REQUIRED/PIECE_LOST.

### 6. Retry / abort limit

Policy: One auto-recovery attempt for isolated loose piece; no repeated chase.

### 7. Verification sau recovery

Destination free; no loose captured piece in board/recovery ROI; no collateral changes.

### 8. Logging bắt buộc

state_id=ST-P50-A02; solution_ref=SOL-ST-P50-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Drop on-grid/off-grid/outside board/onto other piece.

PASS if capture transaction stays incomplete until clearance verified.

### 10. Module ownership / implementation hooks

LoosePieceDetector

PhysicalLocalizer

ManipulationController

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P50-A03 — Wrong piece removed

| State source | ST-P50-A03 — Wrong piece removed |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Robot lấy nhầm quân thay vì captured piece. |
| Source evidence | Unexpected cell disappears. |
| Requirements | R1,R6,R10 |
| Scenario | S10 |

### 1. Possible root causes

Localization targeted neighboring piece.

Board identity/state wrong.

Robot registration error.

Gripper contacted multiple pieces.

### 2. Evidence cần để phân biệt root cause

Expected captured cell still occupied.

Unexpected cell disappears.

ObservedBoard identity before/after.

### 3. Detection / decision logic

Logic đề xuất: Wrong-piece condition must be detected before proceeding with own-piece move.

### 4. Immediate safety / state action

Stop motion at safe hold/clearance; do not place in capture bin.

### 5. Automatic recovery / operator procedure

If wrong piece is securely held, original cell is known/clear and return motion is validated: return it to original cell once, verify.

Otherwise require human intervention.

After restore, revalidate capture target before retry.

### 6. Retry / abort limit

Policy: No repeated wrong-piece retries; one rollback only if unambiguous and safe.

### 7. Verification sau recovery

Wrong piece restored exactly; ExpectedBoard recovered; captured target still known.

No collateral discrepancy.

### 8. Logging bắt buộc

state_id=ST-P50-A03; solution_ref=SOL-ST-P50-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Shift target one cell; corrupt mapping.

PASS if wrong piece never reaches bin and logical move not committed.

### 10. Module ownership / implementation hooks

BoardRecognizer

ManipulationController

RecoveryManager

BoardReconciler

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P50-A04 — Captured piece not cleared from board/bin path

| State source | ST-P50-A04 — Captured piece not cleared from board/bin path |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Quân đã gắp nhưng còn nằm trong vùng bàn hoặc vùng có thể gây nhầm/va chạm. |
| Source evidence | Extra loose object in scene. |
| Requirements | R2,R6,R10,R11 |
| Scenario | Capture/bin anomaly |

### 1. Possible root causes

Piece dropped after pickup but before bin.

Bin/path location inside camera causes loose-object detection.

Release failed so piece remains near board.

### 2. Evidence cần để phân biệt root cause

Loose-object scan across board + configured capture path/out area.

Captured-piece identity from CurrentAction.

Destination cell state.

### 3. Detection / decision logic

Logic đề xuất: Capture removal completes only when destination is clear AND no captured piece remains in prohibited scene zone.

### 4. Immediate safety / state action

Pause transaction; no P60.

### 5. Automatic recovery / operator procedure

Localize loose captured piece.

If isolated/reachable: move to validated bin pose.

If not safely recoverable: HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: One recovery attempt; repeated failure => human.

### 7. Verification sau recovery

Prohibited zone clear; destination clear; no extra object.

Capture sub-transaction marked VERIFIED.

### 8. Logging bắt buộc

state_id=ST-P50-A04; solution_ref=SOL-ST-P50-A04

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Place captured piece near board edge/path; release short of bin.

PASS if system does not continue with loose captured piece present.

### 10. Module ownership / implementation hooks

LoosePieceDetector

RecoveryManager

ManipulationController

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 11. P60 — Pick Moving Piece

Mục tiêu phase: Pick thành công chỉ khi đúng quân thực sự rời source và không tạo side-effect.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P60-N01 SOL-ST-P60-N01 | NORMAL | Moving-piece target localized |
| ST-P60-N02 SOL-ST-P60-N02 | NORMAL | Correct piece grasped |
| ST-P60-T01 SOL-ST-P60-T01 | TRANSIENT / AMBIGUOUS | Piece offset but still inside safe grasp tolerance |
| ST-P60-A01 SOL-ST-P60-A01 | ANOMALY | Miss grasp |
| ST-P60-A02 SOL-ST-P60-A02 | ANOMALY | Wrong adjacent piece picked |
| ST-P60-A03 SOL-ST-P60-A03 | ANOMALY | Two pieces picked / stuck together |
| ST-P60-A04 SOL-ST-P60-A04 | ANOMALY | Pick pushes or shifts neighboring piece |
| ST-P60-A05 SOL-ST-P60-A05 | ANOMALY | Gripper actuator fails to open/close as commanded |
| ST-P60-A06 SOL-ST-P60-A06 | ANOMALY | Unsafe contact / Z-height error during pick |

## SOL-ST-P60-N01 — Moving-piece target localized

| State source | ST-P60-N01 — Moving-piece target localized |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Source piece có physical target đủ tin cậy. |
| Source evidence | continuous XY + valid registration. |
| Requirements | R3,R5,R9 |
| Scenario | S03 |

### Normal handling contract

Xác nhận evidence của state: continuous XY + valid registration..

Chỉ cho phép transition sang: Approach pick.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P60-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P60-N02 — Correct piece grasped

| State source | ST-P60-N02 — Correct piece grasped |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Gripper đã kẹp đúng quân tại source. |
| Source evidence | Source becomes empty and evidence consistent with CurrentAction. |
| Requirements | R6,R9,R10,R13 |
| Scenario | S07 nominal counterpart |

### Normal handling contract

Xác nhận evidence của state: Source becomes empty and evidence consistent with CurrentAction..

Chỉ cho phép transition sang: P70 lift/transport.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P60-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P60-T01 — Piece offset but still inside safe grasp tolerance

| State source | ST-P60-T01 — Piece offset but still inside safe grasp tolerance |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Quân lệch khỏi logical intersection nhưng target vật lý vẫn nằm trong tolerance. |
| Source evidence | offset != 0, uncertainty acceptable. |
| Requirements | R3,R5,R9 |
| Scenario | S03 |

### 1. Possible root causes

Human/previous move left piece off-center but still within tolerance.

Minor perspective/localization offset.

Piece physically shifted without semantic cell change.

### 2. Evidence cần để phân biệt root cause

Continuous XY offset.

Uncertainty bound.

Derived grasp tolerance and neighbor clearance.

### 3. Detection / decision logic

Logic đề xuất: Use visual target only if total error budget stays within safe capture region.

### 4. Immediate safety / state action

Do not use theoretical intersection center blindly when safe visual correction exists.

### 5. Automatic recovery / operator procedure

Generate corrected pick pose from continuous XY.

Execute normal pick under same verification gates.

### 6. Retry / abort limit

Policy: No retry needed solely for offset; if pick fails route A01.

### 7. Verification sau recovery

Correct piece grasp verified; no neighbor motion.

Recorded actual offset for benchmark.

### 8. Logging bắt buộc

state_id=ST-P60-T01; solution_ref=SOL-ST-P60-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Offset 0/2/5/8/10 mm in multiple directions.

PASS success within validated tolerance and no action outside it.

### 10. Module ownership / implementation hooks

PhysicalLocalizer

RobotRegistration

ManipulationController

PostActionVerifier

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P60-A01 — Miss grasp

| State source | ST-P60-A01 — Miss grasp |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Gripper đóng nhưng không lấy được quân. |
| Source evidence | Source piece remains; no carried-piece evidence. |
| Requirements | R9,R10,R13 |
| Scenario | S07 |

### 1. Possible root causes

Localization/registration error.

Wrong Z height/open width.

Gripper timing/slip.

Piece moved during approach.

### 2. Evidence cần để phân biệt root cause

Source still occupied after retract/verification.

Gripper sensor if available.

Target pose/uncertainty/error codes.

### 3. Detection / decision logic

Logic đề xuất: A pick is successful only after physical evidence shows target left source with no wrong-piece delta.

### 4. Immediate safety / state action

Stop before TRANSPORT.

### 5. Automatic recovery / operator procedure

Rescan source, re-localize.

If scene safe and target isolated, retry pick once with corrected target/height.

Second failure => HUMAN_REQUIRED or ROBOT_ERROR if actuator evidence.

### 6. Retry / abort limit

Policy: Default one physical retry after cause-correcting rescan; never infinite.

### 7. Verification sau recovery

Source empty; expected target carried/consistent; no other cell changed.

### 8. Logging bắt buộc

state_id=ST-P60-A01; solution_ref=SOL-ST-P60-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Miss by XY, high Z, no close.

PASS if TRANSPORT never starts on miss.

### 10. Module ownership / implementation hooks

ManipulationController

PostActionVerifier

PhysicalLocalizer

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P60-A02 — Wrong adjacent piece picked

| State source | ST-P60-A02 — Wrong adjacent piece picked |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Robot lấy nhầm quân bên cạnh. |
| Source evidence | Expected source remains, another cell disappears. |
| Requirements | R1,R6,R10 |
| Scenario | S10 |

### 1. Possible root causes

Target XY wrong/registration shifted.

Neighbor too close.

Board state/identity wrong.

### 2. Evidence cần để phân biệt root cause

Expected source remains.

Adjacent/other cell disappears.

Pre/post semantic board.

### 3. Detection / decision logic

Logic đề xuất: Compare disappeared cell(s) to CurrentAction.src immediately after pick.

### 4. Immediate safety / state action

Do not transport to planned destination.

Hold/retract only to a validated safe clearance if already grasping.

### 5. Automatic recovery / operator procedure

If single wrong piece securely held and original cell known+clear: return to original cell, verify.

Then re-localize intended piece.

Any ambiguity/collateral change => HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: One safe rollback; no automatic second attempt until full board resync.

### 7. Verification sau recovery

Wrong piece restored; Expected==Observed; intended source still occupied.

No extra loose piece.

### 8. Logging bắt buộc

state_id=ST-P60-A02; solution_ref=SOL-ST-P60-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Force pick one-cell offset.

PASS if wrong piece detected before planned place.

### 10. Module ownership / implementation hooks

BoardRecognizer

ManipulationController

RecoveryManager

BoardReconciler

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P60-A03 — Two pieces picked / stuck together

| State source | ST-P60-A03 — Two pieces picked / stuck together |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Một thao tác làm hai quân cùng rời bàn. |
| Source evidence | Two expected cells disappear / stacked carry. |
| Requirements | R1,R6,R10,R11 |
| Scenario | Multi-object anomaly |

### 1. Possible root causes

Pieces physically touching/stacked.

Gripper aperture captures neighbor.

Two pieces stuck mechanically.

### 2. Evidence cần để phân biệt root cause

Two logical cells become empty after one pick.

Visual/gripper evidence of stacked carry.

Pre-pick geometry.

### 3. Detection / decision logic

Logic đề xuất: More than one independent disappearance after a single pick is critical multi-object anomaly.

### 4. Immediate safety / state action

Stop further transport toward destination.

Keep/return robot to safe static condition per hardware safety.

### 5. Automatic recovery / operator procedure

Baseline: no autonomous separation on board.

If a prevalidated recovery tray exists and motion is safe, place pair there; otherwise HUMAN_REQUIRED.

Human restores both pieces, then full board verify.

### 6. Retry / abort limit

Policy: No automatic retry.

### 7. Verification sau recovery

Both pieces restored correctly; no stack/loose object; board synchronized.

### 8. Logging bắt buộc

state_id=ST-P60-A03; solution_ref=SOL-ST-P60-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Place touching pieces then pick one.

PASS if system never treats double pick as successful single pick.

### 10. Module ownership / implementation hooks

PostActionVerifier

BoardRecognizer

SafetyManager

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P60-A04 — Pick pushes or shifts neighboring piece

| State source | ST-P60-A04 — Pick pushes or shifts neighboring piece |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Gripper/approach làm xê dịch quân khác dù target có thể đã gắp đúng. |
| Source evidence | Collateral geometry/semantic change. |
| Requirements | R3,R6,R9,R10 |
| Scenario | Collateral movement |

### 1. Possible root causes

Approach path too close to neighbor.

Gripper body/finger collision.

Target piece too offset.

Z/rotation inaccurate.

### 2. Evidence cần để phân biệt root cause

Target may disappear as expected plus neighbor geometry/semantic change.

Pre/post continuous piece centers.

Robot path/pose telemetry.

### 3. Detection / decision logic

Logic đề xuất: Any unplanned neighbor displacement is SIDE_EFFECT even if target pick succeeded.

### 4. Immediate safety / state action

Stop planned transaction before destination place if safe.

Do not commit.

### 5. Automatic recovery / operator procedure

If carried target can be moved to a prevalidated recovery/parking pose safely, do so; otherwise hold/stop per robot safety.

Re-scan board and classify neighbor displacement.

Single, well-known correctable displacement may be restored only after dedicated validated policy; baseline => human correction.

### 6. Retry / abort limit

Policy: No automatic continuation after collateral movement.

### 7. Verification sau recovery

Board restored to Expected pre-action state or explicitly reconciled; carried piece status known.

### 8. Logging bắt buộc

state_id=ST-P60-A04; solution_ref=SOL-ST-P60-A04

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Set neighbor close; force approach collision.

PASS if side-effect detected and no silent completion.

### 10. Module ownership / implementation hooks

ManipulationController

PhysicalLocalizer

BoardReconciler

RecoveryManager

SafetyManager

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P60-A05 — Gripper actuator fails to open/close as commanded

| State source | ST-P60-A05 — Gripper actuator fails to open/close as commanded |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Command kẹp không tạo trạng thái cơ khí mong muốn. |
| Source evidence | gripper error/no movement/feedback mismatch. |
| Requirements | R9,R10,R13 |
| Scenario | Hardware failure |

### 1. Possible root causes

Stepper/servo/gripper driver fault.

I/O/relay command failure.

Power supply issue.

Mechanical jam.

### 2. Evidence cần để phân biệt root cause

Commanded gripper state vs feedback/visual evidence.

I/O state/error code/current sensor if available.

No mechanical movement.

### 3. Detection / decision logic

Logic đề xuất: Treat command sent != actuator success; feedback mismatch is hardware fault.

### 4. Immediate safety / state action

Stop manipulation; inhibit further pick/place.

If robot can retract without worsening contact, use only safe recovery motion; otherwise stop in place and follow hardware procedure.

### 5. Automatic recovery / operator procedure

Do not repeatedly toggle actuator blindly.

Route ST-P110-X03 ROBOT_ERROR; operator diagnoses power/I/O/mechanics.

### 6. Retry / abort limit

Policy: At most one diagnostic command when safe; no auto retry under load.

### 7. Verification sau recovery

Gripper state verified after repair; robot fault cleared per vendor procedure; board re-observed before resume.

### 8. Logging bắt buộc

state_id=ST-P60-A05; solution_ref=SOL-ST-P60-A05

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Disconnect actuator control; jam gripper; I/O mismatch.

PASS if system does not declare grasp success from command alone.

### 10. Module ownership / implementation hooks

GripperDriver

ManipulationController

RobotHealthMonitor

SafetyManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P60-A06 — Unsafe contact / Z-height error during pick

| State source | ST-P60-A06 — Unsafe contact / Z-height error during pick |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | TCP/gripper chạm bàn hoặc quân theo cách không an toàn. |
| Source evidence | contact/collision/abnormal pose. |
| Requirements | R5,R9,R11 |
| Scenario | Physical collision |

### 1. Possible root causes

PICK_Z too low.

TCP/tool frame wrong.

Board height changed.

Registration/Z calibration invalid.

Unexpected obstacle/piece stack.

### 2. Evidence cần để phân biệt root cause

Robot force/contact/fault if available.

Commanded/actual Z.

Geometry/board height version.

Visual collision evidence.

### 3. Detection / decision logic

Logic đề xuất: Unexpected contact or unsafe Z is safety-critical; do not recover by continuing downward/upward sequence automatically.

### 4. Immediate safety / state action

Immediate motion stop via appropriate robot/safety function.

Mark unsafe/robot error depending signal.

### 5. Automatic recovery / operator procedure

Operator inspects scene/hardware.

Revalidate TCP, Z heights, board plane and registration.

Resume only from known safe pose and synchronized board.

### 6. Retry / abort limit

Policy: No autonomous retry after collision/contact.

### 7. Verification sau recovery

Safety fault cleared; calibration verified; dry-run/slow test passes.

### 8. Logging bắt buộc

state_id=ST-P60-A06; solution_ref=SOL-ST-P60-A06

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Set PICK_Z too low; add stacked piece/obstacle.

PASS if stop occurs and no second motion is issued automatically.

### 10. Module ownership / implementation hooks

ManipulationController

RobotHealthMonitor

SafetyManager

RobotRegistration

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 12. P70 — Lift & Transport

Mục tiêu phase: Trong transport phải biết moving_piece ở đâu; drop được phân loại theo vị trí để quyết định auto recovery hay stop.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P70-N01 SOL-ST-P70-N01 | NORMAL | Piece lifted clear of board |
| ST-P70-N02 SOL-ST-P70-N02 | NORMAL | Transport in progress |
| ST-P70-A01 SOL-ST-P70-A01 | ANOMALY | Drop onto an empty valid intersection |
| ST-P70-A02 SOL-ST-P70-A02 | ANOMALY | Drop between intersections / off-grid |
| ST-P70-A03 SOL-ST-P70-A03 | ANOMALY | Drop onto occupied intersection / another piece |
| ST-P70-A04 SOL-ST-P70-A04 | ANOMALY | Drop outside board but inside recovery zone |
| ST-P70-A05 SOL-ST-P70-A05 | ANOMALY | Drop outside camera/workspace |
| ST-P70-A06 SOL-ST-P70-A06 | ANOMALY | Collision causes collateral piece movement |
| ST-P70-T01 SOL-ST-P70-T01 | TRANSIENT / AMBIGUOUS | Carry state uncertain due to occlusion |

## SOL-ST-P70-N01 — Piece lifted clear of board

| State source | ST-P70-N01 — Piece lifted clear of board |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Quân đã được nhấc đủ cao, source verified empty, không kéo theo quân khác. |
| Source evidence | clearance achieved. |
| Requirements | R9,R10,R13 |
| Scenario | Robot move |

### Normal handling contract

Xác nhận evidence của state: clearance achieved..

Chỉ cho phép transition sang: Transport.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P70-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P70-N02 — Transport in progress

| State source | ST-P70-N02 — Transport in progress |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Robot đang mang quân từ source tới destination. |
| Source evidence | CurrentAction phase=TRANSPORT. |
| Requirements | R6,R9,R13 |
| Scenario | Robot move |

### Normal handling contract

Xác nhận evidence của state: CurrentAction phase=TRANSPORT..

Chỉ cho phép transition sang: Approach destination.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P70-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P70-A01 — Drop onto an empty valid intersection

| State source | ST-P70-A01 — Drop onto an empty valid intersection |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Quân rơi vào một giao điểm hợp lệ nhưng không phải destination. |
| Source evidence | Moving piece absent from gripper; extra piece at another legal cell. |
| Requirements | R1,R6,R10 |
| Scenario | S09 |

### 1. Possible root causes

Grip slips during transport.

Unexpected gripper opening/vibration.

Motion acceleration too high.

### 2. Evidence cần để phân biệt root cause

Source empty; planned destination empty.

Moving piece appears at another valid empty intersection.

CurrentAction identifies moving_piece.

### 3. Detection / decision logic

Logic đề xuất: If exactly one extra semantic piece matching CurrentAction appears at an otherwise empty legal cell, classify on-grid drop.

### 4. Immediate safety / state action

Stop normal move; do not commit new cell as game move.

### 5. Automatic recovery / operator procedure

Treat detected cell C as physical recovery source.

Verify C isolated and robot mapping valid.

Pick moving_piece from C -> verify -> continue to original destination or restore according to transaction policy.

Do not update ExpectedBoard to C.

### 6. Retry / abort limit

Policy: One auto-recovery attempt; failure => human.

### 7. Verification sau recovery

C returns to expected prior occupant state (usually empty).

Moving piece verified at intended destination.

Expected==Observed.

### 8. Logging bắt buộc

state_id=ST-P70-A01; solution_ref=SOL-ST-P70-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Drop at several empty intersections.

PASS if system recovers using CurrentAction identity and never commits accidental C position.

### 10. Module ownership / implementation hooks

BoardRecognizer

RecoveryManager

ManipulationController

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P70-A02 — Drop between intersections / off-grid

| State source | ST-P70-A02 — Drop between intersections / off-grid |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Quân rơi ở vị trí không thuộc 90 intersection. |
| Source evidence | Loose object off-grid. |
| Requirements | R2,R3,R5,R10,R13 |
| Scenario | S08 |

### 1. Possible root causes

Grip slips/opening.

Transport collision/vibration.

Piece not centered in grasp.

Gripper force insufficient.

### 2. Evidence cần để phân biệt root cause

Source empty; destination empty.

LoosePieceDetector finds object off-grid.

Continuous XY + uncertainty + recovery-zone status.

### 3. Detection / decision logic

Logic đề xuất: Search free-space only after normal semantic checks indicate moving_piece is missing; do not snap loose piece to nearest intersection.

### 4. Immediate safety / state action

Stop planned placement; maintain safe robot pose.

Mark CurrentAction as DROP_RECOVERY.

### 5. Automatic recovery / operator procedure

Detect loose object -> ensure exactly one plausible target -> localize continuous XY -> check recovery zone/clearance -> repick -> VERIFY_PICK -> transport to original destination -> place -> full board verify.

If object near another piece/uncertainty high, route A03/HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: One loose-piece auto recovery attempt in clean recovery zone; repeated drop => human/robot diagnostic.

### 7. Verification sau recovery

Loose location cleared.

Moving piece at intended destination within geometric tolerance.

No collateral discrepancy.

### 8. Logging bắt buộc

state_id=ST-P70-A02; solution_ref=SOL-ST-P70-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Drop 30+ off-grid points including center/edge.

PASS recovery success per R10 and unsafe recovery rate 0.

### 10. Module ownership / implementation hooks

LoosePieceDetector

PhysicalLocalizer

RobotRegistration

RecoveryManager

ManipulationController

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P70-A03 — Drop onto occupied intersection / another piece

| State source | ST-P70-A03 — Drop onto occupied intersection / another piece |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Quân rơi lên/sát một quân khác, có nguy cơ overlap/stack. |
| Source evidence | Overlapping/touching objects; ambiguous physical state. |
| Requirements | R2,R10,R11,R12 |
| Scenario | S18 |

### 1. Possible root causes

Piece drops directly on another piece.

Bounce causes touching/overlap.

Existing destination/neighbor occupied unexpectedly.

### 2. Evidence cần để phân biệt root cause

Overlapping/touching contours/masks.

Semantic board ambiguous.

Depth/height abnormal if available.

### 3. Detection / decision logic

Logic đề xuất: If loose target cannot be separated/localized with safe clearance, mark unsafe ambiguity; do not guess which center to grasp.

### 4. Immediate safety / state action

Stop autonomous recovery/motion near stack.

### 5. Automatic recovery / operator procedure

Baseline policy = HUMAN_REQUIRED.

Human separates/restores pieces using displayed expected positions.

Then full board rescan/reconcile.

### 6. Retry / abort limit

Policy: No auto-retry for stacked/overlap state unless a separately validated manipulation strategy exists.

### 7. Verification sau recovery

No overlap; all piece identities/positions verified; board sync restored.

### 8. Logging bắt buộc

state_id=ST-P70-A03; solution_ref=SOL-ST-P70-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Drop onto occupied cell; partial overlap/touching.

PASS if auto recovery is inhibited.

### 10. Module ownership / implementation hooks

LoosePieceDetector

SafetyManager

RecoveryManager

BoardReconciler

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P70-A04 — Drop outside board but inside recovery zone

| State source | ST-P70-A04 — Drop outside board but inside recovery zone |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Quân rơi ngoài board ROI nhưng camera thấy và robot có thể với tới theo định nghĩa recovery zone. |
| Source evidence | Loose object in recovery ROI. |
| Requirements | R2,R3,R5,R10 |
| Scenario | S11 |

### 1. Possible root causes

Piece slips beyond board boundary but remains within camera and robot recovery ROI.

Bounce/trajectory carries piece off board.

### 2. Evidence cần để phân biệt root cause

Loose object center.

Recovery-zone classification.

Reachability/clearance and board mapping validity.

### 3. Detection / decision logic

Logic đề xuất: Outside board is recoverable only if target lies inside explicitly configured camera+robot recovery zone and scene is safe.

### 4. Immediate safety / state action

Stop normal transaction; no game-state commit.

### 5. Automatic recovery / operator procedure

Localize loose piece in recovery coordinate frame.

Reachability/safety preflight.

Repick -> verify -> move to original destination -> verify full board.

If not reachable => P40-A01 / HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: One auto-recovery attempt.

### 7. Verification sau recovery

Loose object removed from recovery ROI.

Moving piece at destination; no board disturbance.

### 8. Logging bắt buộc

state_id=ST-P70-A04; solution_ref=SOL-ST-P70-A04

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Drop at several off-board positions inside/outside recovery polygon.

PASS only inside configured zone gets auto recovery.

### 10. Module ownership / implementation hooks

LoosePieceDetector

PhysicalLocalizer

RobotRegistration

RecoveryManager

SafetyManager

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P70-A05 — Drop outside camera/workspace

| State source | ST-P70-A05 — Drop outside camera/workspace |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Moving piece biến mất và không thể localize trong vùng cho phép. |
| Source evidence | No object found; destination empty; source empty. |
| Requirements | R2,R10,R12,R13 |
| Scenario | S12 |

### 1. Possible root causes

Piece falls outside camera FOV.

Falls below table/behind object.

Loose detector fails despite in-FOV object.

Workspace cannot reach visible piece.

### 2. Evidence cần để phân biệt root cause

Source empty, destination empty.

Full recovery ROI scan returns no valid target.

Camera health valid to exclude sensor failure.

### 3. Detection / decision logic

Logic đề xuất: Only declare PIECE_LOST after camera/geometry health pass and exhaustive configured recovery scan finds no plausible object.

### 4. Immediate safety / state action

No autonomous motion searching blindly.

Enter ST-P110-X05.

### 5. Automatic recovery / operator procedure

Human locates/replaces moving piece.

System rescans board and reconciles ExpectedBoard/CurrentAction.

Resume only from a known board state.

### 6. Retry / abort limit

Policy: No auto retry beyond one full search/rescan.

### 7. Verification sau recovery

Piece physically restored to agreed cell or move completed manually; full board sync verified.

CurrentAction resolved explicitly.

### 8. Logging bắt buộc

state_id=ST-P70-A05; solution_ref=SOL-ST-P70-A05

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Remove moving piece out of FOV during simulated transport.

PASS if system stops and asks human, never guesses location.

### 10. Module ownership / implementation hooks

LoosePieceDetector

CameraHealthMonitor

RecoveryManager

StateCoordinator

UI

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P70-A06 — Collision causes collateral piece movement

| State source | ST-P70-A06 — Collision causes collateral piece movement |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Robot/gripper/quân đang mang va vào bàn/quân khác làm scene thay đổi ngoài kế hoạch. |
| Source evidence | Multiple unexpected geometry/semantic changes. |
| Requirements | R6,R9,R10,R11 |
| Scenario | S17 |

### 1. Possible root causes

Transport path too low/close to pieces.

Carried piece swings/slips.

Unexpected board movement/obstacle.

Trajectory/planning error.

### 2. Evidence cần để phân biệt root cause

Robot fault/contact if available.

Multiple changed cells after motion.

Trajectory and pose telemetry.

Before/after board geometry.

### 3. Detection / decision logic

Logic đề xuất: Collision + multiple unexpected deltas is complex safety anomaly, not single-piece recovery.

### 4. Immediate safety / state action

Immediate stop/inhibit further motion according to safety system.

Do not place/commit.

### 5. Automatic recovery / operator procedure

After safe stop, full board rescan and discrepancy classification.

If robot still holds piece, handle only via predefined safe hold/parking procedure.

Baseline complex multi-change => HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: No automatic retry after collision.

### 7. Verification sau recovery

Human/validated procedure restores scene; board sync and geometry/registration verified before resume.

### 8. Logging bắt buộc

state_id=ST-P70-A06; solution_ref=SOL-ST-P70-A06

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Transport low over neighboring pieces; inject collision.

PASS if no auto continuation and multiple deltas logged.

### 10. Module ownership / implementation hooks

SafetyManager

ManipulationController

BoardReconciler

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P70-T01 — Carry state uncertain due to occlusion

| State source | ST-P70-T01 — Carry state uncertain due to occlusion |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Camera không xác định được robot còn giữ quân hay không trong một đoạn transport. |
| Source evidence | No direct visual evidence; action still in progress. |
| Requirements | R7,R11,R12,R13 |
| Scenario | S19 |

### 1. Possible root causes

Overhead camera line-of-sight blocked by gripper/arm.

Piece fully hidden during transport.

### 2. Evidence cần để phân biệt root cause

CurrentAction=TRANSPORT.

Expected self-occlusion interval.

Gripper feedback if available.

Next camera-clear checkpoint.

### 3. Detection / decision logic

Logic đề xuất: Absence of visual evidence during expected occlusion is not evidence of drop.

### 4. Immediate safety / state action

Do not update board from occluded frames.

Continue only along already-approved transport path if safety state remains valid.

### 5. Automatic recovery / operator procedure

At first camera-clear checkpoint, verify carried-piece evidence or final destination/source state.

If verification fails, classify into specific drop/miss state.

### 6. Retry / abort limit

Policy: No rescan while robot physically blocks view unless it can stop at a prevalidated observation pose.

### 7. Verification sau recovery

After clear, carried status/post-condition can be verified.

No false drop event during occlusion.

### 8. Logging bắt buộc

state_id=ST-P70-T01; solution_ref=SOL-ST-P70-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Cover piece with gripper during lift/transport.

PASS if no false failure and later verification still required.

### 10. Module ownership / implementation hooks

SceneStabilityGate

RobotStateAdapter

PostActionVerifier

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 13. P80 — Place at Destination

Mục tiêu phase: Place không chỉ đúng logical cell mà còn phải ổn định về geometry.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P80-N01 SOL-ST-P80-N01 | NORMAL | Destination approach valid |
| ST-P80-N02 SOL-ST-P80-N02 | NORMAL | Piece released and stable at destination |
| ST-P80-A01 SOL-ST-P80-A01 | ANOMALY | Piece placed with excessive offset |
| ST-P80-A02 SOL-ST-P80-A02 | ANOMALY | Piece sticks to gripper after release |
| ST-P80-A03 SOL-ST-P80-A03 | ANOMALY | Piece tips / lies down |
| ST-P80-A04 SOL-ST-P80-A04 | ANOMALY | Destination unexpectedly occupied |
| ST-P80-A05 SOL-ST-P80-A05 | ANOMALY | Place pushes neighboring piece |
| ST-P80-A06 SOL-ST-P80-A06 | ANOMALY | Piece released at wrong logical cell |

## SOL-ST-P80-N01 — Destination approach valid

| State source | ST-P80-N01 — Destination approach valid |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Robot tới vùng trên destination với pose/clearance hợp lệ. |
| Source evidence | destination known/reachable/safe. |
| Requirements | R5,R9,R11 |
| Scenario | Robot move |

### Normal handling contract

Xác nhận evidence của state: destination known/reachable/safe..

Chỉ cho phép transition sang: Lower & place.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P80-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P80-N02 — Piece released and stable at destination

| State source | ST-P80-N02 — Piece released and stable at destination |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Quân được nhả tại destination, đứng ổn định. |
| Source evidence | Physical piece present at expected destination. |
| Requirements | R9,R10 |
| Scenario | Robot move |

### Normal handling contract

Xác nhận evidence của state: Physical piece present at expected destination..

Chỉ cho phép transition sang: Retract then verify.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P80-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P80-A01 — Piece placed with excessive offset

| State source | ST-P80-A01 — Piece placed with excessive offset |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Semantic cell có thể đúng nhưng tâm vật lý lệch quá tolerance. |
| Source evidence | Correct logical cell + geometric offset unsafe. |
| Requirements | R3,R9,R10 |
| Scenario | S03 variant |

### 1. Possible root causes

Destination localization/registration error.

Release while piece swings.

Piece rolls/slides after release.

Target semantic cell correct but physical tolerance exceeded.

### 2. Evidence cần để phân biệt root cause

Continuous center after release.

Distance to intended intersection in mm.

Neighbor clearance.

### 3. Detection / decision logic

Logic đề xuất: Semantic correctness is insufficient; geometric offset > safe threshold => place failure.

### 4. Immediate safety / state action

Do not commit move yet.

### 5. Automatic recovery / operator procedure

If piece isolated and regrasp target safe: repick once and re-center at destination.

If near neighbor/uncertain => HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: One corrective re-place attempt maximum.

### 7. Verification sau recovery

Logical destination correct AND physical offset within tolerance.

No collateral movement.

### 8. Logging bắt buộc

state_id=ST-P80-A01; solution_ref=SOL-ST-P80-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Release at controlled offsets.

PASS if excessive offset prevents P100 commit.

### 10. Module ownership / implementation hooks

PhysicalLocalizer

ManipulationController

PostActionVerifier

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P80-A02 — Piece sticks to gripper after release

| State source | ST-P80-A02 — Piece sticks to gripper after release |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Gripper mở nhưng quân vẫn dính/được kéo lên theo gripper. |
| Source evidence | Destination remains empty or piece moves with gripper. |
| Requirements | R9,R10 |
| Scenario | Place anomaly |

### 1. Possible root causes

Static friction/magnetism/adhesion.

Gripper not fully opening.

Piece wedged in fingers.

Retract starts too quickly.

### 2. Evidence cần để phân biệt root cause

Destination empty after release command.

Piece moves with gripper in successive frames.

Gripper feedback.

### 3. Detection / decision logic

Logic đề xuất: Release command success requires visual/sensor proof piece remains at destination when gripper retracts.

### 4. Immediate safety / state action

Stop/reverse retract before moving far if safely possible.

Do not mark PLACE_OK.

### 5. Automatic recovery / operator procedure

Return to a validated release height above destination.

Open/close-clear/open sequence at most once if hardware-safe.

Slow retract and verify.

If persists => ROBOT_ERROR/HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: One release retry; repeated sticking -> no further auto cycles.

### 7. Verification sau recovery

Piece stationary at destination; gripper clear/open; board verified.

### 8. Logging bắt buộc

state_id=ST-P80-A02; solution_ref=SOL-ST-P80-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Simulate sticky piece or incomplete open.

PASS if destination empty triggers failure rather than commit.

### 10. Module ownership / implementation hooks

GripperDriver

ManipulationController

PostActionVerifier

RecoveryManager

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P80-A03 — Piece tips / lies down

| State source | ST-P80-A03 — Piece tips / lies down |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Quân ở destination nhưng không ở tư thế gắp/chơi bình thường. |
| Source evidence | Abnormal contour/height/orientation. |
| Requirements | R2,R3,R10 |
| Scenario | S18 related |

### 1. Possible root causes

Release height too high.

Piece edge contact/uneven surface.

Piece geometry damaged.

Collision during release.

### 2. Evidence cần để phân biệt root cause

Abnormal contour/orientation/height.

Piece center + shape.

Destination semantic identity if recognizable.

### 3. Detection / decision logic

Logic đề xuất: A tipped/lying piece is physical anomaly even if center maps to correct cell.

### 4. Immediate safety / state action

Do not continue game/commit if next grasp may be unsafe.

### 5. Automatic recovery / operator procedure

Baseline: request human to upright piece.

Optional auto regrasp/upright only after separately validated manipulation routine.

After correction, re-verify geometry and semantic board.

### 6. Retry / abort limit

Policy: No baseline auto retry.

### 7. Verification sau recovery

Piece upright/stable and within position tolerance.

### 8. Logging bắt buộc

state_id=ST-P80-A03; solution_ref=SOL-ST-P80-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Place from increasing release heights; intentionally tip piece.

PASS if tipped state blocks normal progression.

### 10. Module ownership / implementation hooks

PhysicalLocalizer

Shape/HeightChecker

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P80-A04 — Destination unexpectedly occupied

| State source | ST-P80-A04 — Destination unexpectedly occupied |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Tại thời điểm place, destination không còn trống như plan. |
| Source evidence | Observed occupancy conflict. |
| Requirements | R1,R6,R11,R12 |
| Scenario | State changed unexpectedly |

### 1. Possible root causes

Human/another process changed board during robot move.

Capture piece removal failed.

ExpectedBoard stale.

Perception false occupancy.

### 2. Evidence cần để phân biệt root cause

Fresh destination snapshot.

Expected destination occupancy.

CurrentAction/capture status.

Confidence and scene stability.

### 3. Detection / decision logic

Logic đề xuất: Destination must be revalidated shortly before descent/release; occupied conflict aborts place.

### 4. Immediate safety / state action

Stop descent/place; keep piece in safe held state if possible.

No release onto occupied cell.

### 5. Automatic recovery / operator procedure

Rescan board.

If capture-removal incomplete, route P50 recovery.

If unexpected human/state change, reconcile/human.

Use recovery/parking pose only if prevalidated.

### 6. Retry / abort limit

Policy: No retry until board sync and destination explicitly free.

### 7. Verification sau recovery

Destination free and expected state reconciled; carried piece status known.

### 8. Logging bắt buộc

state_id=ST-P80-A04; solution_ref=SOL-ST-P80-A04

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Place another piece at destination during transport; capture removal miss.

PASS if robot never releases onto occupied intersection.

### 10. Module ownership / implementation hooks

PostActionVerifier

BoardRecognizer

BoardReconciler

ManipulationController

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P80-A05 — Place pushes neighboring piece

| State source | ST-P80-A05 — Place pushes neighboring piece |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Thao tác hạ/nhả làm quân khác dịch chuyển. |
| Source evidence | Collateral change near destination. |
| Requirements | R6,R9,R10 |
| Scenario | Collateral movement |

### 1. Possible root causes

Descent/release path too close to neighbor.

Piece/gripper offset.

Neighbor already off-center.

### 2. Evidence cần để phân biệt root cause

Changed neighboring center/cell.

Target placement result.

Robot pose and clearance.

### 3. Detection / decision logic

Logic đề xuất: Any unexpected neighbor displacement after place = SIDE_EFFECT.

### 4. Immediate safety / state action

No commit; retract to safe pose if possible.

### 5. Automatic recovery / operator procedure

Full local/full-board rescan.

If only geometric small shift and an auto-centering routine has been validated, it may be corrected; baseline => human correction.

Then verify moving piece and neighbor.

### 6. Retry / abort limit

Policy: No blind repeat place.

### 7. Verification sau recovery

All affected pieces back within semantic/geometric tolerance.

### 8. Logging bắt buộc

state_id=ST-P80-A05; solution_ref=SOL-ST-P80-A05

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Neighbor close to destination; intentionally push.

PASS if collateral movement detected.

### 10. Module ownership / implementation hooks

PhysicalLocalizer

BoardReconciler

RecoveryManager

ManipulationController

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P80-A06 — Piece released at wrong logical cell

| State source | ST-P80-A06 — Piece released at wrong logical cell |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Quân được đặt ổn định nhưng vào giao điểm khác destination. |
| Source evidence | Observed piece at wrong cell. |
| Requirements | R1,R6,R10 |
| Scenario | Robot placement error |

### 1. Possible root causes

Wrong destination pose due registration/indexing bug.

Coordinate row/col swapped/orientation error.

Trajectory target stale.

### 2. Evidence cần để phân biệt root cause

Moving piece observed at wrong logical cell.

CurrentAction.dst.

Commanded robot target/board coordinate.

### 3. Detection / decision logic

Logic đề xuất: Compare final cell against immutable CurrentAction.dst, not latest mutable UI state.

### 4. Immediate safety / state action

No commit; mark wrong-place.

### 5. Automatic recovery / operator procedure

If wrong cell isolated and piece identity known: re-localize there, pick once, place at intended dst, verify.

Before retry validate orientation/mapping; if mapping suspect route P40-A02.

If ambiguity => human.

### 6. Retry / abort limit

Policy: One corrective move after root cause validation.

### 7. Verification sau recovery

Wrong cell restored to prior expected content; destination contains moving piece; full board synchronized.

### 8. Logging bắt buộc

state_id=ST-P80-A06; solution_ref=SOL-ST-P80-A06

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Swap row/col in test target; command adjacent cell.

PASS wrong cell never gets committed as intended move.

### 10. Module ownership / implementation hooks

BoardRecognizer

StateStore

RobotRegistration

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 14. P90 — Post-action Verification

Mục tiêu phase: Verification là gate bắt buộc trước commit; symptom được chẩn đoán thành state cụ thể.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P90-N01 SOL-ST-P90-N01 | NORMAL | Post-action board verified |
| ST-P90-T01 SOL-ST-P90-T01 | TRANSIENT / AMBIGUOUS | Verification temporarily blocked |
| ST-P90-A01 SOL-ST-P90-A01 | ANOMALY | Destination empty after supposed place |
| ST-P90-A02 SOL-ST-P90-A02 | ANOMALY | Source still occupied after supposed pick/move |
| ST-P90-A03 SOL-ST-P90-A03 | ANOMALY | Wrong cell/piece changed |
| ST-P90-A04 SOL-ST-P90-A04 | ANOMALY | Multiple independent discrepancies |
| ST-P90-A05 SOL-ST-P90-A05 | ANOMALY | Semantic board correct but physical geometry unsafe |
| ST-P90-A06 SOL-ST-P90-A06 | ANOMALY | Verification confidence insufficient |

## SOL-ST-P90-N01 — Post-action board verified

| State source | ST-P90-N01 — Post-action board verified |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | ObservedBoard khớp expected post-condition của CurrentAction. |
| Source evidence | Expected == observed; no extra loose piece. |
| Requirements | R1,R6,R10,R12,R13 |
| Scenario | E2E nominal |

### Normal handling contract

Xác nhận evidence của state: Expected == observed; no extra loose piece..

Chỉ cho phép transition sang: P100 commit.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Expected post-condition == ObservedBoard đã verify.

Không có loose piece/extra discrepancy.

Cho phép P100 commit.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P90-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P90-T01 — Verification temporarily blocked

| State source | ST-P90-T01 — Verification temporarily blocked |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Robot/tay người/blur khiến snapshot verify chưa đủ tin cậy. |
| Source evidence | occlusion/blur/low confidence. |
| Requirements | R7,R11,R12,R13 |
| Scenario | S14,S19 |

### 1. Possible root causes

Robot still occluding board.

Motion blur/autofocus settling.

Human entered scene.

Camera exposure settling.

### 2. Evidence cần để phân biệt root cause

Robot_clear state.

Blur/quality metric.

Occlusion/hand signal.

Frame freshness.

### 3. Detection / decision logic

Logic đề xuất: Verification snapshot must be taken only after scene-stable gate; blocked snapshot is not failure proof.

### 4. Immediate safety / state action

Do not commit or launch next move.

### 5. Automatic recovery / operator procedure

Wait for robot clear/scene stable; discard stale buffer; take fresh burst.

If unable to obtain valid observation within limit -> ST-P90-A06 or CAMERA_INVALID.

### 6. Retry / abort limit

Policy: 2 fresh verification attempts after scene clear.

### 7. Verification sau recovery

Valid fresh snapshot; confidence passes; post-condition decisively verified or specific anomaly identified.

### 8. Logging bắt buộc

state_id=ST-P90-T01; solution_ref=SOL-ST-P90-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Blur immediately after retract; robot blocks camera.

PASS if system waits instead of reading invalid frame.

### 10. Module ownership / implementation hooks

SceneStabilityGate

CameraMonitor

PostActionVerifier

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P90-A01 — Destination empty after supposed place

| State source | ST-P90-A01 — Destination empty after supposed place |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Software/robot nghĩ đã place nhưng destination không có quân. |
| Source evidence | destination empty. |
| Requirements | R1,R6,R10 |
| Scenario | E2E-03 risk |

### 1. Possible root causes

Piece stayed on gripper (P80-A02).

Piece dropped during/after place.

Robot never actually carried piece due miss pick.

Perception false negative at destination.

### 2. Evidence cần để phân biệt root cause

Destination burst + confidence.

Source occupancy.

Loose-piece scan.

Gripper/carry evidence/current action history.

### 3. Detection / decision logic

Logic đề xuất: Use diagnostic routing: dst empty alone is symptom, not root cause.

### 4. Immediate safety / state action

No commit.

### 5. Automatic recovery / operator procedure

Rescan destination/source.

If source occupied -> route P90-A02/P60-A01.

If piece on gripper/stuck -> P80-A02.

If loose piece found -> P70 drop recovery.

If no piece found -> PIECE_LOST.

### 6. Retry / abort limit

Policy: One diagnostic rescan plus state-specific recovery limit.

### 7. Verification sau recovery

Moving piece location resolved and intended post-condition verified.

No unresolved loose object.

### 8. Logging bắt buộc

state_id=ST-P90-A01; solution_ref=SOL-ST-P90-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Create each root cause with same dst-empty symptom.

PASS if diagnostic routing picks evidence-consistent state rather than arbitrary retry.

### 10. Module ownership / implementation hooks

PostActionVerifier

BoardRecognizer

LoosePieceDetector

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P90-A02 — Source still occupied after supposed pick/move

| State source | ST-P90-A02 — Source still occupied after supposed pick/move |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Source vẫn còn quân, cho thấy pick/move không diễn ra như kỳ vọng. |
| Source evidence | source occupied. |
| Requirements | R1,R6,R10 |
| Scenario | S07 |

### 1. Possible root causes

Miss grasp.

Piece briefly moved then returned/slipped back.

Perception wrongly thought source cleared earlier.

### 2. Evidence cần để phân biệt root cause

Fresh source snapshot.

Destination/loose scan.

Pick verification event history.

### 3. Detection / decision logic

Logic đề xuất: Source occupied after supposed completed move invalidates post-condition regardless of robot API success.

### 4. Immediate safety / state action

No commit.

### 5. Automatic recovery / operator procedure

If destination empty and source piece intact: treat as miss grasp -> controlled retry from P60 after re-localization.

If other discrepancies exist -> complex anomaly/human.

### 6. Retry / abort limit

Policy: One pick retry if clean single failure.

### 7. Verification sau recovery

Source empty after verified pick; destination final correct.

### 8. Logging bắt buộc

state_id=ST-P90-A02; solution_ref=SOL-ST-P90-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Force gripper miss but SDK success.

PASS silent physical failure = 0.

### 10. Module ownership / implementation hooks

PostActionVerifier

RecoveryManager

ManipulationController

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P90-A03 — Wrong cell/piece changed

| State source | ST-P90-A03 — Wrong cell/piece changed |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | ObservedBoard cho thấy một cell khác hoặc piece identity khác bị tác động. |
| Source evidence | unexpected delta. |
| Requirements | R1,R6,R10 |
| Scenario | S10 |

### 1. Possible root causes

Wrong piece picked.

Wrong destination.

Collateral push.

Semantic recognition error.

### 2. Evidence cần để phân biệt root cause

Unexpected delta list with identities.

CurrentAction src/dst/piece.

Confidence across affected cells.

### 3. Detection / decision logic

Logic đề xuất: Any delta outside expected post-condition must block commit.

### 4. Immediate safety / state action

No commit; stop autonomous continuation.

### 5. Automatic recovery / operator procedure

Rescan to confirm.

Route to wrong-pick, wrong-place, or collateral-specific recovery if exactly classified.

If not uniquely classifiable => HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: No generic retry until root cause classified.

### 7. Verification sau recovery

Expected post-condition restored exactly; no extra delta.

### 8. Logging bắt buộc

state_id=ST-P90-A03; solution_ref=SOL-ST-P90-A03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Pick adjacent piece; place at wrong cell; push neighbor.

PASS if unexpected delta always blocks commit.

### 10. Module ownership / implementation hooks

PostActionVerifier

BoardReconciler

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P90-A04 — Multiple independent discrepancies

| State source | ST-P90-A04 — Multiple independent discrepancies |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Có từ hai mismatch độc lập trở lên sau action. |
| Source evidence | discrepancy set size > 1. |
| Requirements | R6,R10,R12,R13 |
| Scenario | S17 |

### 1. Possible root causes

Collision/multi-piece disturbance.

Human interference during robot action.

Perception/geometry catastrophic failure.

Compound failures.

### 2. Evidence cần để phân biệt root cause

Full discrepancy set >=2 independent changes.

Scene/safety events.

CurrentAction logs.

Geometry/camera health.

### 3. Detection / decision logic

Logic đề xuất: Do not assume single root cause or perform greedy one-by-one auto repairs.

### 4. Immediate safety / state action

Stop auto recovery; safe robot state.

### 5. Automatic recovery / operator procedure

One full rescan/geometry check to rule out perception artifact.

If still multiple physical discrepancies => HUMAN_REQUIRED.

Human restores/reconciles using ExpectedBoard diff.

### 6. Retry / abort limit

Policy: No autonomous multi-anomaly repair baseline.

### 7. Verification sau recovery

Discrepancy set empty and board/geometry valid after intervention.

### 8. Logging bắt buộc

state_id=ST-P90-A04; solution_ref=SOL-ST-P90-A04

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Disturb 2-3 pieces; combine drop + collision.

PASS if system escalates rather than loops recoveries.

### 10. Module ownership / implementation hooks

PostActionVerifier

BoardReconciler

SafetyManager

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P90-A05 — Semantic board correct but physical geometry unsafe

| State source | ST-P90-A05 — Semantic board correct but physical geometry unsafe |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Logical state đúng nhưng một quân lệch/chạm/overlap ở mức có thể gây thất bại lượt sau. |
| Source evidence | semantic equal; geometry invalid. |
| Requirements | R2,R3,R10 |
| Scenario | S03,S18 |

### 1. Possible root causes

Placed piece too off-center.

Piece tipped/touching neighbor.

Neighbor shifted but semantic cells unchanged.

### 2. Evidence cần để phân biệt root cause

Continuous centers, shape/overlap.

Semantic board equality.

Tolerance budget.

### 3. Detection / decision logic

Logic đề xuất: Verification must include physical geometry for pieces affected by action, not semantic matrix only.

### 4. Immediate safety / state action

No commit if geometry can compromise next action/safety.

### 5. Automatic recovery / operator procedure

If exactly one isolated piece merely off-center and safe re-center routine validated: corrective pick/place once.

Tipped/overlap/complex geometry => human.

### 6. Retry / abort limit

Policy: One corrective geometry action max.

### 7. Verification sau recovery

Semantic board correct AND geometric constraints pass.

### 8. Logging bắt buộc

state_id=ST-P90-A05; solution_ref=SOL-ST-P90-A05

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Offset/tip/touch cases with same semantic labels.

PASS if semantic-equal unsafe geometry is caught.

### 10. Module ownership / implementation hooks

PhysicalLocalizer

PostActionVerifier

RecoveryManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P90-A06 — Verification confidence insufficient

| State source | ST-P90-A06 — Verification confidence insufficient |
| --- | --- |
| Type / Severity | ANOMALY / Major |
| Source definition | Không có đủ evidence để khẳng định action thành công hoặc thất bại. |
| Source evidence | UNKNOWN/low confidence. |
| Requirements | R1,R12,R13 |
| Scenario | S14 |

### 1. Possible root causes

Low-light/blur/occlusion.

Model disagreement.

Geometry residual high.

Camera health degrading.

### 2. Evidence cần để phân biệt root cause

All confidence scores/residuals.

Frame quality/freshness.

Temporal consistency.

### 3. Detection / decision logic

Logic đề xuất: Insufficient evidence must return UNKNOWN; no successful action decision without post-condition proof.

### 4. Immediate safety / state action

No commit; robot remains inhibited for next physical action.

### 5. Automatic recovery / operator procedure

Fresh burst after scene clear.

If geometry/camera invalid route corresponding terminal state.

If repeated perception ambiguity only => HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: 2 verification rescans; then escalate.

### 7. Verification sau recovery

Either VERIFIED or a specific failure state established with sufficient evidence.

### 8. Logging bắt buộc

state_id=ST-P90-A06; solution_ref=SOL-ST-P90-A06

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Blur, glare, partial cover after place.

PASS if UNKNOWN never maps to success.

### 10. Module ownership / implementation hooks

PostActionVerifier

ConfidencePolicy

CameraHealthMonitor

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 15. P100 — Commit & Turn Handover

Mục tiêu phase: Commit atomic, single-writer, chỉ sau VERIFIED.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P100-N01 SOL-ST-P100-N01 | NORMAL | Move committed and turn advanced |
| ST-P100-N02 SOL-ST-P100-N02 | NORMAL | Board synchronized for next turn |
| ST-P100-A01 SOL-ST-P100-A01 | ANOMALY | Commit attempted with unresolved discrepancy |
| ST-P100-A02 SOL-ST-P100-A02 | ANOMALY | State update race / bypass coordinator |

## SOL-ST-P100-N01 — Move committed and turn advanced

| State source | ST-P100-N01 — Move committed and turn advanced |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Verified physical result được commit vào logical state và chuyển lượt. |
| Source evidence | state_version increments; CurrentAction completes. |
| Requirements | R6,R8,R13 |
| Scenario | S01,S02 |

### Normal handling contract

Xác nhận evidence của state: state_version increments; CurrentAction completes..

Chỉ cho phép transition sang: Back to P10.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P100-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P100-N02 — Board synchronized for next turn

| State source | ST-P100-N02 — Board synchronized for next turn |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Sau commit, ExpectedBoard == ObservedBoard và system sẵn sàng lượt mới. |
| Source evidence | No discrepancy. |
| Requirements | R6,R13 |
| Scenario | E2E-04 |

### Normal handling contract

Xác nhận evidence của state: No discrepancy..

Chỉ cho phép transition sang: P10 stable.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P100-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P100-A01 — Commit attempted with unresolved discrepancy

| State source | ST-P100-A01 — Commit attempted with unresolved discrepancy |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Một module cố update state dù P90 chưa VERIFIED. |
| Source evidence | unresolved discrepancy + commit event. |
| Requirements | R6,R13 |
| Scenario | R13 hard failure |

### 1. Possible root causes

Coordinator guard missing.

Async callback commits early.

A module writes GameState directly.

### 2. Evidence cần để phân biệt root cause

P90 status != VERIFIED.

Commit event trace.

discrepancy set.

state transition log.

### 3. Detection / decision logic

Logic đề xuất: Commit function must assert post-condition verified and discrepancy_set empty.

### 4. Immediate safety / state action

Reject commit atomically; keep prior state_version.

Raise workflow invariant event.

### 5. Automatic recovery / operator procedure

Route back to P90/recovery based on unresolved state.

If code bug repeatedly triggers, safe-stop current game and flag engineering fault.

### 6. Retry / abort limit

Policy: No commit retry until P90 VERIFIED.

### 7. Verification sau recovery

No state_version increment on rejected commit.

After proper verify, single atomic commit succeeds.

### 8. Logging bắt buộc

state_id=ST-P100-A01; solution_ref=SOL-ST-P100-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Call commit API with unresolved discrepancy.

PASS invalid commit rejected 100%.

### 10. Module ownership / implementation hooks

StateCoordinator

TransactionManager

StateStore

PostActionVerifier

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P100-A02 — State update race / bypass coordinator

| State source | ST-P100-A02 — State update race / bypass coordinator |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Nhiều module update board/action không theo một coordinator duy nhất. |
| Source evidence | inconsistent state_version / half commit. |
| Requirements | R6,R13 |
| Scenario | Software architecture failure |

### 1. Possible root causes

Multiple threads write board state.

Direct writes bypass coordinator.

Non-atomic persistence.

Server/local update race.

### 2. Evidence cần để phân biệt root cause

state_version gaps/duplicates.

Event timestamps/writer identity.

Half-updated ExpectedBoard/CurrentAction.

### 3. Detection / decision logic

Logic đề xuất: Only one coordinator may mutate authoritative state; compare-and-swap/version guard rejects stale writers.

### 4. Immediate safety / state action

Freeze further state mutations and physical actions.

Persist diagnostic snapshot.

### 5. Automatic recovery / operator procedure

Reload last durable consistent version/event log.

Reconstruct board from physical observation.

Do not resume until writer architecture/config fixed or race proven transient.

Runtime baseline => HUMAN_REQUIRED/restart-safe path.

### 6. Retry / abort limit

Policy: No auto retry of conflicting writes.

### 7. Verification sau recovery

State invariants hold; single writer confirmed; physical/logical board reconciled.

### 8. Logging bắt buộc

state_id=ST-P100-A02; solution_ref=SOL-ST-P100-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Concurrent simulated writers; stale state_version update.

PASS half commit = 0 and stale update rejected.

### 10. Module ownership / implementation hooks

StateCoordinator

StateStore

TransactionManager

ConcurrencyGuard

────────────────────────────────────────────────────────────────────────────────

# 16. P110 — Recovery / Safe Terminal States

Mục tiêu phase: Recovery có giới hạn và phải kết thúc bằng VERIFIED hoặc safe terminal.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P110-N01 SOL-ST-P110-N01 | NORMAL | Rescan in progress |
| ST-P110-N02 SOL-ST-P110-N02 | NORMAL | Controlled retry in progress |
| ST-P110-N03 SOL-ST-P110-N03 | NORMAL | Loose-piece recovery in progress |
| ST-P110-T01 SOL-ST-P110-T01 | TRANSIENT / AMBIGUOUS | Recovery result ambiguous |
| ST-P110-X01 SOL-ST-P110-X01 | TERMINAL / SAFE STOP | HUMAN_REQUIRED |
| ST-P110-X02 SOL-ST-P110-X02 | TERMINAL / SAFE STOP | CAMERA_INVALID |
| ST-P110-X03 SOL-ST-P110-X03 | TERMINAL / SAFE STOP | ROBOT_ERROR |
| ST-P110-X04 SOL-ST-P110-X04 | TERMINAL / SAFE STOP | UNSAFE_STOP |
| ST-P110-X05 SOL-ST-P110-X05 | TERMINAL / SAFE STOP | PIECE_LOST |

## SOL-ST-P110-N01 — Rescan in progress

| State source | ST-P110-N01 — Rescan in progress |
| --- | --- |
| Type / Severity | NORMAL / Recovery |
| Source definition | Hệ thống chủ động quan sát lại để phân giải UNKNOWN/AMBIGUOUS. |
| Source evidence | new frame/burst requested. |
| Requirements | R1,R7,R12,R13 |
| Scenario | General recovery |

### Normal handling contract

Xác nhận evidence của state: new frame/burst requested..

Chỉ cho phép transition sang: Return to originating verification state.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Recovery procedure kết thúc bằng một state cụ thể, không để UNKNOWN âm thầm.

Quay về state gốc để verify hoặc escalate terminal nếu không đạt.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P110-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P110-N02 — Controlled retry in progress

| State source | ST-P110-N02 — Controlled retry in progress |
| --- | --- |
| Type / Severity | NORMAL / Recovery |
| Source definition | Một physical action được thử lại trong retry limit. |
| Source evidence | attempt counter increments. |
| Requirements | R9,R10,R13 |
| Scenario | S07 |

### Normal handling contract

Xác nhận evidence của state: attempt counter increments..

Chỉ cho phép transition sang: Re-verify.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Recovery procedure kết thúc bằng một state cụ thể, không để UNKNOWN âm thầm.

Quay về state gốc để verify hoặc escalate terminal nếu không đạt.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P110-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P110-N03 — Loose-piece recovery in progress

| State source | ST-P110-N03 — Loose-piece recovery in progress |
| --- | --- |
| Type / Severity | NORMAL / Recovery |
| Source definition | Hệ thống đã xác định loose piece trong recovery zone và đang thực hiện recovery transaction. |
| Source evidence | target localized + recovery attempt. |
| Requirements | R2,R3,R5,R10,R13 |
| Scenario | S08,S11 |

### Normal handling contract

Xác nhận evidence của state: target localized + recovery attempt..

Chỉ cho phép transition sang: Re-verify board.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Recovery procedure kết thúc bằng một state cụ thể, không để UNKNOWN âm thầm.

Quay về state gốc để verify hoặc escalate terminal nếu không đạt.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P110-N03.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P110-T01 — Recovery result ambiguous

| State source | ST-P110-T01 — Recovery result ambiguous |
| --- | --- |
| Type / Severity | TRANSIENT / AMBIGUOUS / Warning |
| Source definition | Recovery action kết thúc nhưng board vẫn chưa xác định chắc. |
| Source evidence | uncertainty remains. |
| Requirements | R10,R12,R13 |
| Scenario | S17 |

### 1. Possible root causes

Recovery altered scene but verification still low-confidence.

More than one piece could match target.

Recovery caused secondary displacement.

Camera/geometry uncertainty.

### 2. Evidence cần để phân biệt root cause

Post-recovery board + discrepancy set.

Recovery attempt log/count.

Confidence/geometry residual.

### 3. Detection / decision logic

Logic đề xuất: An ambiguous recovery outcome is not success and must not trigger another blind physical retry.

### 4. Immediate safety / state action

Stop further autonomous recovery actions.

### 5. Automatic recovery / operator procedure

One fresh rescan/geometry health check.

If ambiguity resolves to a specific recoverable state, route there.

If not -> HUMAN_REQUIRED.

### 6. Retry / abort limit

Policy: No second physical recovery until state becomes unambiguous.

### 7. Verification sau recovery

Final state either VERIFIED or explicit safe terminal; never left half-resolved.

### 8. Logging bắt buộc

state_id=ST-P110-T01; solution_ref=SOL-ST-P110-T01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Recovery target near neighbor; low-confidence post recovery.

PASS if retry loop terminates.

### 10. Module ownership / implementation hooks

RecoveryManager

PostActionVerifier

ConfidencePolicy

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P110-X01 — HUMAN_REQUIRED

| State source | ST-P110-X01 — HUMAN_REQUIRED |
| --- | --- |
| Type / Severity | TERMINAL / SAFE STOP / Terminal |
| Source definition | Hệ thống dừng auto-action và yêu cầu người can thiệp. |
| Source evidence | No further autonomous motion. |
| Requirements | R10,R12,R13 |
| Scenario | S12,S17,S18 |

### 1. Possible root causes

Retry limit reached.

Ambiguous/complex state.

No validated auto-recovery path.

Operator decision required.

### 2. Evidence cần để phân biệt root cause

Triggering state ID.

Full Expected/Observed diff.

CurrentAction and attempt count.

Evidence images/logs.

### 3. Detection / decision logic

Logic đề xuất: Terminal means no autonomous physical action until explicit human resolution and resume.

### 4. Immediate safety / state action

Inhibit robot motion; keep/command safe pose only through validated safety procedure.

Persist full state snapshot.

### 5. Automatic recovery / operator procedure

UI shows exact State ID, symptom, expected vs observed cells and recommended manual correction.

After human correction: explicit RESUME -> full camera/geometry/board validation -> new transaction.

### 6. Retry / abort limit

Policy: No automatic exit from HUMAN_REQUIRED.

### 7. Verification sau recovery

Operator acknowledges; board/robot/camera safe; Expected==Observed; no active unknown action.

### 8. Logging bắt buộc

state_id=ST-P110-X01; solution_ref=SOL-ST-P110-X01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Trigger via retry limit, overlap, multi-anomaly.

PASS if no autonomous motion occurs while X01 active.

### 10. Module ownership / implementation hooks

StateCoordinator

RecoveryManager

SafetyManager

UI

StateStore

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P110-X02 — CAMERA_INVALID

| State source | ST-P110-X02 — CAMERA_INVALID |
| --- | --- |
| Type / Severity | TERMINAL / SAFE STOP / Terminal |
| Source definition | Camera/geometry không đủ tin cậy để tự động tiếp tục. |
| Source evidence | camera health invalid. |
| Requirements | R11,R12,R13 |
| Scenario | S14 |

### 1. Possible root causes

Camera disconnected/frozen.

Geometry cannot be trusted.

Repeated capture failure.

### 2. Evidence cần để phân biệt root cause

Camera health status/errors.

Last fresh frame time.

Geometry validity.

### 3. Detection / decision logic

Logic đề xuất: CAMERA_INVALID blocks any perception-dependent robot action.

### 4. Immediate safety / state action

Stop/inhibit motion and persist action state.

### 5. Automatic recovery / operator procedure

Operator/controlled service reconnects camera.

Then warm-up -> fresh-frame test -> geometry calibration/validation -> full board rescan/reconcile.

Only explicit coordinator transition may exit X02.

### 6. Retry / abort limit

Policy: No automatic repeated camera reset loop.

### 7. Verification sau recovery

Camera fresh; geometry valid; board trusted/synchronized.

### 8. Logging bắt buộc

state_id=ST-P110-X02; solution_ref=SOL-ST-P110-X02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Disconnect camera in each workflow phase.

PASS if no action continues using stale imagery.

### 10. Module ownership / implementation hooks

CameraHealthMonitor

GeometryManager

SafetyManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P110-X03 — ROBOT_ERROR

| State source | ST-P110-X03 — ROBOT_ERROR |
| --- | --- |
| Type / Severity | TERMINAL / SAFE STOP / Terminal |
| Source definition | Robot/gripper/controller báo lỗi không thể tiếp tục action tự động. |
| Source evidence | robot fault state. |
| Requirements | R9,R10,R13 |
| Scenario | Hardware failure |

### 1. Possible root causes

Robot controller fault.

Gripper/IO/driver fault.

Protective stop or hardware error.

Motion command exception with unsafe unknown pose.

### 2. Evidence cần để phân biệt root cause

Robot/controller error code.

Actual pose if available.

Gripper feedback/IO.

CurrentAction phase.

### 3. Detection / decision logic

Logic đề xuất: Software must not auto-clear safety/controller faults unless explicitly supported and validated by vendor procedure.

### 4. Immediate safety / state action

Stop issuing motion commands; preserve native protective stop/E-stop semantics.

Persist CurrentAction.

### 5. Automatic recovery / operator procedure

Operator diagnoses/clears fault using robot procedure.

After recovery: home/safe pose only if allowed, revalidate tool/TCP/registration and board state.

Reconcile incomplete CurrentAction.

### 6. Retry / abort limit

Policy: No autonomous restart after hardware fault.

### 7. Verification sau recovery

Robot health READY; safety cleared; board/registration synchronized.

### 8. Logging bắt buộc

state_id=ST-P110-X03; solution_ref=SOL-ST-P110-X03

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Inject SDK/controller error; gripper fault.

PASS if system remains stopped until explicit recovery.

### 10. Module ownership / implementation hooks

RobotHealthMonitor

ManipulationController

SafetyManager

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P110-X04 — UNSAFE_STOP

| State source | ST-P110-X04 — UNSAFE_STOP |
| --- | --- |
| Type / Severity | TERMINAL / SAFE STOP / Terminal |
| Source definition | Safety gate phát hiện người/vật cản/collision/unknown nguy hiểm. |
| Source evidence | safety state != SAFE. |
| Requirements | R11,R12,R13 |
| Scenario | S15 |

### 1. Possible root causes

Human/obstacle in safety zone.

Collision/contact.

Safety state UNKNOWN.

Native safety trigger.

### 2. Evidence cần để phân biệt root cause

Safety inputs/events.

Human/obstacle evidence.

Robot protective stop status.

### 3. Detection / decision logic

Logic đề xuất: Any safety-critical signal dominates game logic; no recovery movement unless allowed by safety system/procedure.

### 4. Immediate safety / state action

Immediate inhibit/stop via safety-rated mechanisms available; application stops issuing commands.

### 5. Automatic recovery / operator procedure

Human clears hazard.

Operator acknowledges/resets safety as required.

Revalidate camera/geometry/board and incomplete action before resume.

### 6. Retry / abort limit

Policy: No auto resume just because vision later says clear.

### 7. Verification sau recovery

Native safety state ready, scene SAFE for debounce interval, explicit resume, board/action reconciled.

### 8. Logging bắt buộc

state_id=ST-P110-X04; solution_ref=SOL-ST-P110-X04

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Hand enters motion zone; collision trigger.

PASS unsafe auto-action = 0.

### 10. Module ownership / implementation hooks

SafetyManager

RobotSafetyInterface

StateCoordinator

UI

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P110-X05 — PIECE_LOST

| State source | ST-P110-X05 — PIECE_LOST |
| --- | --- |
| Type / Severity | TERMINAL / SAFE STOP / Terminal |
| Source definition | Hệ thống biết một quân phải tồn tại nhưng không thể tìm thấy trong camera/workspace. |
| Source evidence | source empty, dst empty, no loose object found. |
| Requirements | R2,R6,R10,R12 |
| Scenario | S12 |

### 1. Possible root causes

Dropped piece outside FOV/workspace.

Piece hidden under/behind object.

Detector failure after exhaustive healthy scan.

### 2. Evidence cần để phân biệt root cause

CurrentAction moving_piece.

Source/dst empty.

Recovery scan result.

Camera/geometry health PASS.

### 3. Detection / decision logic

Logic đề xuất: Declare PIECE_LOST only after ruling out camera invalid and known in-zone drop locations.

### 4. Immediate safety / state action

No search motion into unknown areas.

Freeze game state.

### 5. Automatic recovery / operator procedure

Human finds/replaces exact moving piece.

UI provides expected identity/src/dst/current phase.

After manual recovery, full board reconstruction and explicit action resolution.

### 6. Retry / abort limit

Policy: No autonomous exit.

### 7. Verification sau recovery

All 90 cells + recovery ROI consistent; moving piece accounted for; CurrentAction closed/recovered.

### 8. Logging bắt buộc

state_id=ST-P110-X05; solution_ref=SOL-ST-P110-X05

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Physically remove moving piece from FOV.

PASS if human intervention is required and state remains auditable.

### 10. Module ownership / implementation hooks

RecoveryManager

BoardReconciler

UI

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 17. P120 — Game End / Shutdown / Restart

Mục tiêu phase: Shutdown/restart phải bảo toàn transaction và reconstruct physical state trước resume.

| State/Solution | Type | Short name |
| --- | --- | --- |
| ST-P120-N01 SOL-ST-P120-N01 | NORMAL | Game over recognized |
| ST-P120-N02 SOL-ST-P120-N02 | NORMAL | Clean shutdown |
| ST-P120-A01 SOL-ST-P120-A01 | ANOMALY | Software crash mid-action |
| ST-P120-N03 SOL-ST-P120-N03 | NORMAL | Restart on non-initial board |
| ST-P120-A02 SOL-ST-P120-A02 | ANOMALY | Persisted state and physical board disagree after restart |

## SOL-ST-P120-N01 — Game over recognized

| State source | ST-P120-N01 — Game over recognized |
| --- | --- |
| Type / Severity | NORMAL / Terminal-Normal |
| Source definition | Luật game xác nhận ván đã kết thúc; không lập kế hoạch move tiếp. |
| Source evidence | game_over true. |
| Requirements | R8,R13 |
| Scenario | Game end |

### Normal handling contract

Xác nhận evidence của state: game_over true..

Chỉ cho phép transition sang: Shutdown/idle.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P120-N01.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P120-N02 — Clean shutdown

| State source | ST-P120-N02 — Clean shutdown |
| --- | --- |
| Type / Severity | NORMAL / Terminal-Normal |
| Source definition | Hệ thống dừng có kiểm soát, persist state cần thiết. |
| Source evidence | No active physical transaction. |
| Requirements | R6,R13 |
| Scenario | Shutdown |

### Normal handling contract

Xác nhận evidence của state: No active physical transaction..

Chỉ cho phép transition sang: Off.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P120-N02.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P120-A01 — Software crash mid-action

| State source | ST-P120-A01 — Software crash mid-action |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | Process dừng/exception trong lúc CurrentAction chưa hoàn tất. |
| Source evidence | persisted action incomplete / physical state unknown. |
| Requirements | R6,R10,R13 |
| Scenario | S16 |

### 1. Possible root causes

Unhandled exception/process kill.

Power/network loss.

Dependency crash while physical transaction active.

### 2. Evidence cần để phân biệt root cause

Crash log/exception.

Last persisted CurrentAction and event sequence.

Last robot/gripper state if available.

### 3. Detection / decision logic

Logic đề xuất: Persist action phase before each important physical side-effect so restart can detect incomplete transaction.

### 4. Immediate safety / state action

On process death no further app action; on restart enter P00-A04, not normal game loop.

### 5. Automatic recovery / operator procedure

Restart in safe mode.

Read persisted state -> observe physical board -> classify source/dst/loose/carry state -> reconcile.

Resume only if a validated recovery path proves physical state; otherwise human.

### 6. Retry / abort limit

Policy: No automatic continuation from the next command in old sequence.

### 7. Verification sau recovery

Crash recovery preserves state invariants; board/action reconciled; no duplicate side-effect.

### 8. Logging bắt buộc

state_id=ST-P120-A01; solution_ref=SOL-ST-P120-A01

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Kill process after each phase boundary.

PASS no crash point produces silent full commit or duplicated move.

### 10. Module ownership / implementation hooks

StateStore

TransactionManager

CrashRecovery

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P120-N03 — Restart on non-initial board

| State source | ST-P120-N03 — Restart on non-initial board |
| --- | --- |
| Type / Severity | NORMAL / Normal |
| Source definition | Ứng dụng restart trong một ván đang chơi và cần reconstruct/sync với physical board. |
| Source evidence | board not initial setup. |
| Requirements | R1,R6,R8,R13 |
| Scenario | S20 |

### Normal handling contract

Xác nhận evidence của state: board not initial setup..

Chỉ cho phép transition sang: P00/P10 restore path.

Ghi state_id, state_version, CurrentAction và evidence tối thiểu vào log.

### Exit / verification criteria

Không có discrepancy/UNKNOWN critical liên quan state.

Transition tiếp theo được StateCoordinator cho phép.

### Test tối thiểu

Replay/execute nominal path dẫn tới ST-P120-N03.

PASS nếu state được nhận đúng, không có transition trái phép và workflow đi đúng nhánh tiếp theo.

Log phải có state_id, state_version, frame/action correlation_id tương ứng.

────────────────────────────────────────────────────────────────────────────────

## SOL-ST-P120-A02 — Persisted state and physical board disagree after restart

| State source | ST-P120-A02 — Persisted state and physical board disagree after restart |
| --- | --- |
| Type / Severity | ANOMALY / Critical |
| Source definition | State store và camera reconstruct khác nhau sau restart. |
| Source evidence | Expected vs observed mismatch. |
| Requirements | R1,R6,R10,R12,R13 |
| Scenario | S16,S20 |

### 1. Possible root causes

Physical board changed while app offline.

Persist write lost/old.

Crash occurred after physical side-effect but before logical commit.

Perception/reconstruction error.

### 2. Evidence cần để phân biệt root cause

Persisted ExpectedBoard/version.

Fresh ObservedBoard + confidence.

Incomplete action/event log.

Geometry validity.

### 3. Detection / decision logic

Logic đề xuất: Persistent vs physical mismatch after restart requires reconcile; persisted state is not automatically authoritative.

### 4. Immediate safety / state action

No autonomous motion/turn advancement.

### 5. Automatic recovery / operator procedure

Rescan burst.

Use event log/CurrentAction to explain mismatch.

If unique known crash side-effect -> recover transaction.

Otherwise human chooses/restores canonical board; then persist new verified state.

### 6. Retry / abort limit

Policy: 2 rescan max; unresolved mismatch => HUMAN_REQUIRED.

### 7. Verification sau recovery

ExpectedBoard == verified ObservedBoard; no incomplete transaction; state_version coherent.

### 8. Logging bắt buộc

state_id=ST-P120-A02; solution_ref=SOL-ST-P120-A02

trial_id/correlation_id; timestamp; state_version

ExpectedBoard + ObservedBoard + discrepancy_set liên quan

CurrentAction {piece, src, dst, phase, attempt}

frame_id(s), geometry_version, registration_version nếu liên quan

decision path: ALLOW/RESCAN/RETRY/RECOVER/ABORT/HUMAN_REQUIRED

result + final_state_id

### 9. Test cases + PASS/FAIL

Change board while app off; crash after place before commit.

PASS if restart does not overwrite physical reality or blindly trust persisted state.

### 10. Module ownership / implementation hooks

CrashRecovery

BoardReconciler

StateStore

StateCoordinator

────────────────────────────────────────────────────────────────────────────────

# 18. Traceability: State -> Requirement -> Module -> Test

Mỗi bug/experiment phải log State ID. Từ State ID có thể tìm ngược requirement và solution; từ solution phải có test tái lập.

| State | Solution | R liên quan | Scenario | Implementation owner |
| --- | --- | --- | --- | --- |
| ST-P00-N01 | SOL-ST-P00-N01 | R4,R6,R11,R12,R13 | S20 nếu restart midgame | StateCoordinator + phase module |
| ST-P00-N02 | SOL-ST-P00-N02 | R6,R13 | S20 | StateCoordinator + phase module |
| ST-P00-T01 | SOL-ST-P00-T01 | R1,R4,R12,R13 | Startup transient | CameraHealthMonitor, GeometryManager |
| ST-P00-A01 | SOL-ST-P00-A01 | R11,R12,R13 | S14 | CameraHealthMonitor, CameraMonitor |
| ST-P00-A02 | SOL-ST-P00-A02 | R4,R5,R11,R12 | S13 | GeometryManager, RobotRegistration |
| ST-P00-A03 | SOL-ST-P00-A03 | R1,R6,R8,R12 | S20 | BoardRecognizer, StateStore |
| ST-P00-A04 | SOL-ST-P00-A04 | R6,R10,R13 | S16 | StateStore, TransactionManager |
| ST-P10-N01 | SOL-ST-P10-N01 | R1,R4,R6,R11,R12,R13 | S01,S02 | StateCoordinator + phase module |
| ST-P10-T01 | SOL-ST-P10-T01 | R1,R7,R11,R12 | S04,S19 | SceneStabilityGate, SafetyManager |
| ST-P10-T02 | SOL-ST-P10-T02 | R7,R11,R13 | S19 | RobotStateAdapter, SceneStabilityGate |
| ST-P10-T03 | SOL-ST-P10-T03 | R1,R12,R13 | S14 | BoardRecognizer, ConfidencePolicy |
| ST-P10-A01 | SOL-ST-P10-A01 | R4,R5,R11,R13 | S13 | GeometryManager, RobotRegistration |
| ST-P10-A02 | SOL-ST-P10-A02 | R11,R12,R13 | S14 | CameraHealthMonitor, CameraMonitor |
| ST-P10-A03 | SOL-ST-P10-A03 | R1,R6,R12,R13 | S17,S20 | BoardReconciler, StateStore |
| ST-P20-N01 | SOL-ST-P20-N01 | R6,R7,R13 | S01 | StateCoordinator + phase module |
| ST-P20-T01 | SOL-ST-P20-T01 | R7,R11,R12 | S04 | TemporalReasoner, SceneStabilityGate |
| ST-P20-T02 | SOL-ST-P20-T02 | R7,R11,R12,R13 | Human delayed placement | SceneStabilityGate, TemporalReasoner |
| ST-P20-N02 | SOL-ST-P20-N02 | R1,R6,R7 | S01 | StateCoordinator + phase module |
| ST-P20-N03 | SOL-ST-P20-N03 | R1,R6,R7 | S02 | StateCoordinator + phase module |
| ST-P20-A01 | SOL-ST-P20-A01 | R1,R7,R8,R12 | S05 | RuleEngine, TemporalReasoner |
| ST-P20-A02 | SOL-ST-P20-A02 | R1,R6,R7,R12 | S06,S17 | BoardReconciler, TemporalReasoner |
| ST-P20-A03 | SOL-ST-P20-A03 | R1,R6,R7,R8 | Cheating/error family | BoardRecognizer, BoardReconciler |
| ST-P20-A04 | SOL-ST-P20-A04 | R1,R6,R7,R8 | Cheating/error family | BoardRecognizer, LoosePieceDetector |
| ST-P20-A05 | SOL-ST-P20-A05 | R2,R3,R6,R7 | Off-grid human error | LoosePieceDetector, PhysicalLocalizer |
| ST-P20-A06 | SOL-ST-P20-A06 | R7,R11,R12,R13 | S14/S15 related | SceneStabilityGate, CameraHealthMonitor |
| ST-P30-N01 | SOL-ST-P30-N01 | R6,R7,R12 | S01,S02 | StateCoordinator + phase module |
| ST-P30-N02 | SOL-ST-P30-N02 | R6,R8,R13 | S01,S02 | StateCoordinator + phase module |
| ST-P30-T01 | SOL-ST-P30-T01 | R7,R8,R12 | S17 | TemporalReasoner, RuleEngine |
| ST-P30-T02 | SOL-ST-P30-T02 | R1,R7,R8,R12 | S14 | BoardRecognizer, TemporalReasoner |
| ST-P30-A01 | SOL-ST-P30-A01 | R6,R8,R12 | S05 | RuleEngine, BoardReconciler |
| ST-P30-A02 | SOL-ST-P30-A02 | R6,R7,R8 | S16,S20 | StateStore, BoardReconciler |
| ST-P30-A03 | SOL-ST-P30-A03 | R4,R8,R12 | Orientation failure | GeometryManager, BoardRecognizer |
| ST-P40-N01 | SOL-ST-P40-N01 | R5,R8,R9,R11,R13 | Normal robot move | StateCoordinator + phase module |
| ST-P40-N02 | SOL-ST-P40-N02 | R6,R8,R9,R13 | S02 | StateCoordinator + phase module |
| ST-P40-T01 | SOL-ST-P40-T01 | R3,R12,R13 | S03 | PhysicalLocalizer, RobotRegistration |
| ST-P40-A01 | SOL-ST-P40-A01 | R5,R9,R11,R12 | S12 | RobotRegistration, MotionPlanner |
| ST-P40-A02 | SOL-ST-P40-A02 | R4,R5,R11,R13 | S13 | GeometryManager, RobotRegistration |
| ST-P40-A03 | SOL-ST-P40-A03 | R11,R12,R13 | S15 | SafetyManager, SceneStabilityGate |
| ST-P50-N01 | SOL-ST-P50-N01 | R3,R5,R9 | S02 | StateCoordinator + phase module |
| ST-P50-N02 | SOL-ST-P50-N02 | R6,R9,R10,R13 | S02 | StateCoordinator + phase module |
| ST-P50-A01 | SOL-ST-P50-A01 | R9,R10,R13 | Capture failure | ManipulationController, PostActionVerifier |
| ST-P50-A02 | SOL-ST-P50-A02 | R2,R3,R10 | Drop family | LoosePieceDetector, PhysicalLocalizer |
| ST-P50-A03 | SOL-ST-P50-A03 | R1,R6,R10 | S10 | BoardRecognizer, ManipulationController |
| ST-P50-A04 | SOL-ST-P50-A04 | R2,R6,R10,R11 | Capture/bin anomaly | LoosePieceDetector, RecoveryManager |
| ST-P60-N01 | SOL-ST-P60-N01 | R3,R5,R9 | S03 | StateCoordinator + phase module |
| ST-P60-N02 | SOL-ST-P60-N02 | R6,R9,R10,R13 | S07 nominal counterpart | StateCoordinator + phase module |
| ST-P60-T01 | SOL-ST-P60-T01 | R3,R5,R9 | S03 | PhysicalLocalizer, RobotRegistration |
| ST-P60-A01 | SOL-ST-P60-A01 | R9,R10,R13 | S07 | ManipulationController, PostActionVerifier |
| ST-P60-A02 | SOL-ST-P60-A02 | R1,R6,R10 | S10 | BoardRecognizer, ManipulationController |
| ST-P60-A03 | SOL-ST-P60-A03 | R1,R6,R10,R11 | Multi-object anomaly | PostActionVerifier, BoardRecognizer |
| ST-P60-A04 | SOL-ST-P60-A04 | R3,R6,R9,R10 | Collateral movement | ManipulationController, PhysicalLocalizer |
| ST-P60-A05 | SOL-ST-P60-A05 | R9,R10,R13 | Hardware failure | GripperDriver, ManipulationController |
| ST-P60-A06 | SOL-ST-P60-A06 | R5,R9,R11 | Physical collision | ManipulationController, RobotHealthMonitor |
| ST-P70-N01 | SOL-ST-P70-N01 | R9,R10,R13 | Robot move | StateCoordinator + phase module |
| ST-P70-N02 | SOL-ST-P70-N02 | R6,R9,R13 | Robot move | StateCoordinator + phase module |
| ST-P70-A01 | SOL-ST-P70-A01 | R1,R6,R10 | S09 | BoardRecognizer, RecoveryManager |
| ST-P70-A02 | SOL-ST-P70-A02 | R2,R3,R5,R10,R13 | S08 | LoosePieceDetector, PhysicalLocalizer |
| ST-P70-A03 | SOL-ST-P70-A03 | R2,R10,R11,R12 | S18 | LoosePieceDetector, SafetyManager |
| ST-P70-A04 | SOL-ST-P70-A04 | R2,R3,R5,R10 | S11 | LoosePieceDetector, PhysicalLocalizer |
| ST-P70-A05 | SOL-ST-P70-A05 | R2,R10,R12,R13 | S12 | LoosePieceDetector, CameraHealthMonitor |
| ST-P70-A06 | SOL-ST-P70-A06 | R6,R9,R10,R11 | S17 | SafetyManager, ManipulationController |
| ST-P70-T01 | SOL-ST-P70-T01 | R7,R11,R12,R13 | S19 | SceneStabilityGate, RobotStateAdapter |
| ST-P80-N01 | SOL-ST-P80-N01 | R5,R9,R11 | Robot move | StateCoordinator + phase module |
| ST-P80-N02 | SOL-ST-P80-N02 | R9,R10 | Robot move | StateCoordinator + phase module |
| ST-P80-A01 | SOL-ST-P80-A01 | R3,R9,R10 | S03 variant | PhysicalLocalizer, ManipulationController |
| ST-P80-A02 | SOL-ST-P80-A02 | R9,R10 | Place anomaly | GripperDriver, ManipulationController |
| ST-P80-A03 | SOL-ST-P80-A03 | R2,R3,R10 | S18 related | PhysicalLocalizer, Shape/HeightChecker |
| ST-P80-A04 | SOL-ST-P80-A04 | R1,R6,R11,R12 | State changed unexpectedly | PostActionVerifier, BoardRecognizer |
| ST-P80-A05 | SOL-ST-P80-A05 | R6,R9,R10 | Collateral movement | PhysicalLocalizer, BoardReconciler |
| ST-P80-A06 | SOL-ST-P80-A06 | R1,R6,R10 | Robot placement error | BoardRecognizer, StateStore |
| ST-P90-N01 | SOL-ST-P90-N01 | R1,R6,R10,R12,R13 | E2E nominal | StateCoordinator + phase module |
| ST-P90-T01 | SOL-ST-P90-T01 | R7,R11,R12,R13 | S14,S19 | SceneStabilityGate, CameraMonitor |
| ST-P90-A01 | SOL-ST-P90-A01 | R1,R6,R10 | E2E-03 risk | PostActionVerifier, BoardRecognizer |
| ST-P90-A02 | SOL-ST-P90-A02 | R1,R6,R10 | S07 | PostActionVerifier, RecoveryManager |
| ST-P90-A03 | SOL-ST-P90-A03 | R1,R6,R10 | S10 | PostActionVerifier, BoardReconciler |
| ST-P90-A04 | SOL-ST-P90-A04 | R6,R10,R12,R13 | S17 | PostActionVerifier, BoardReconciler |
| ST-P90-A05 | SOL-ST-P90-A05 | R2,R3,R10 | S03,S18 | PhysicalLocalizer, PostActionVerifier |
| ST-P90-A06 | SOL-ST-P90-A06 | R1,R12,R13 | S14 | PostActionVerifier, ConfidencePolicy |
| ST-P100-N01 | SOL-ST-P100-N01 | R6,R8,R13 | S01,S02 | StateCoordinator + phase module |
| ST-P100-N02 | SOL-ST-P100-N02 | R6,R13 | E2E-04 | StateCoordinator + phase module |
| ST-P100-A01 | SOL-ST-P100-A01 | R6,R13 | R13 hard failure | StateCoordinator, TransactionManager |
| ST-P100-A02 | SOL-ST-P100-A02 | R6,R13 | Software architecture failure | StateCoordinator, StateStore |
| ST-P110-N01 | SOL-ST-P110-N01 | R1,R7,R12,R13 | General recovery | StateCoordinator + phase module |
| ST-P110-N02 | SOL-ST-P110-N02 | R9,R10,R13 | S07 | StateCoordinator + phase module |
| ST-P110-N03 | SOL-ST-P110-N03 | R2,R3,R5,R10,R13 | S08,S11 | StateCoordinator + phase module |
| ST-P110-T01 | SOL-ST-P110-T01 | R10,R12,R13 | S17 | RecoveryManager, PostActionVerifier |
| ST-P110-X01 | SOL-ST-P110-X01 | R10,R12,R13 | S12,S17,S18 | StateCoordinator, RecoveryManager |
| ST-P110-X02 | SOL-ST-P110-X02 | R11,R12,R13 | S14 | CameraHealthMonitor, GeometryManager |
| ST-P110-X03 | SOL-ST-P110-X03 | R9,R10,R13 | Hardware failure | RobotHealthMonitor, ManipulationController |
| ST-P110-X04 | SOL-ST-P110-X04 | R11,R12,R13 | S15 | SafetyManager, RobotSafetyInterface |
| ST-P110-X05 | SOL-ST-P110-X05 | R2,R6,R10,R12 | S12 | RecoveryManager, BoardReconciler |
| ST-P120-N01 | SOL-ST-P120-N01 | R8,R13 | Game end | StateCoordinator + phase module |
| ST-P120-N02 | SOL-ST-P120-N02 | R6,R13 | Shutdown | StateCoordinator + phase module |
| ST-P120-A01 | SOL-ST-P120-A01 | R6,R10,R13 | S16 | StateStore, TransactionManager |
| ST-P120-N03 | SOL-ST-P120-N03 | R1,R6,R8,R13 | S20 | StateCoordinator + phase module |
| ST-P120-A02 | SOL-ST-P120-A02 | R1,R6,R10,R12,R13 | S16,S20 | CrashRecovery, BoardReconciler |

# 19. Recommended implementation order

| Order | Module/Focus | Definition of done |
| --- | --- | --- |
| Step 1 | StateCoordinator + StateStore | Enforce single writer, CurrentAction transaction, state_version và commit guards. |
| Step 2 | CameraHealth + SceneStability + Geometry validity | Để mọi solution có evidence đáng tin và biết khi nào không được motion. |
| Step 3 | PostActionVerifier | Triển khai PROC-09 để loại silent failure trước khi viết recovery phức tạp. |
| Step 4 | Simple recoveries | Miss grasp, rescan, single off-grid drop trong clean recovery zone. |
| Step 5 | Wrong-piece / wrong-place rollback | Chỉ sau khi semantic board + verification đủ chắc. |
| Step 6 | Complex recovery gates | Overlap, collision, multi-anomaly chủ yếu safe-stop/human baseline. |
| Step 7 | Crash/restart reconcile | Persist transaction và test crash injection ở mọi phase. |
| Step 8 | Acceptance matrix | Chạy S01-S20 + từng SOL state injection; freeze thresholds/config khi benchmark. |

# 20. Exit Criteria cho Solution Catalog v1

Mọi State ID trong State Catalog có đúng một SOL-ST-... entry trong tài liệu này.

T/A/X state có root cause, evidence, detection logic, immediate action, recovery, retry/abort, verification, logs và tests.

Normal state có handling contract và exit criteria.

Runtime không có silent physical success, illegal commit hoặc unsafe auto-action trong acceptance suite.

Retry limit và threshold được version hóa; không thay sau khi xem benchmark mà không ghi reason.

Các solution liên quan geometry/robot/gripper được cập nhật sau Phase 0 tolerance measurement.

Mọi recovery phải quay lại PROC-09 Full Post-action Verify trước P100 commit.

# Kết luận

State Catalog trả lời “đang xảy ra vấn đề gì”; Solution & Recovery Catalog trả lời “phải chứng minh nguyên nhân bằng evidence nào, dừng ở đâu, có được tự sửa không, sửa như thế nào và xác nhận thành công ra sao”. Hai tài liệu dùng cùng State ID để team có thể debug, viết issue, code recovery và test mà không lệch ngữ nghĩa.